Testing of NH3/H2 and NH3/syngas Combustion Mechanisms
Using a Large Amount of Experimental Data

A. Gy. Szanthoffer, I. Gy. Zsély, L. Kawka, M. Papp, T. Turányi

Institute of Chemistry, ELTE Eötvös Loránd University

(Pázmány Péter sétány 1/A, 1117, Budapest, Hungary)

# Supplementary Material

Contents

1.	Details of the utilized experimental data	2

2.	Overview of excluded experimental data	25

3.	Simulation of ST-IDT experiments	27

4.	Estimation of the standard deviation of the experimental data	28

4.1.	Ignition delay time measurements	29

4.2.	Concentration measurements	30

4.3.	Laminar burning velocity measurements	33

5.	Investigated reaction mechanisms	36

6.	Quantitative mechanism comparison	40

6.1.	NH3/H2 fuel mixtures	40

7.	Additional sensitivity analysis results	42

7.1.	NH3/H2 fuel mixtures	42

7.2.	NH3/syngas fuel mixtures	44

References	47

# Details of the utilized experimental data

Details of the utilized experimental data are summarized in tables along with estimated overall standard deviations for the data points. These experimental data are available in the ReSpecTh database (http://respecth.hu/)1 in RKD format XML data files. Each row in the tables contains the unique identifier of the XML data file in which the corresponding dataset is stored (XML ID), the source of the experiment (Reference), and the number of data points in the one or more data series contained in the dataset (Dp.). In the case of concentration measurements, it is common that more than one data series belong to a dataset (XML data file) because the concentrations of more than one species were measured in the same experiment. In addition to that, information about the experimental conditions (temperature, T; pressure, p; equivalence ratio, φ; composition of the initial gas mixture) is also shown for each dataset. In the case of concentration measurements (JSR and FR experiments), the species whose concentrations were measured are also shown (Meas.). In the last column of each row (St. dev.), the standard deviation is shown for the data points in the corresponding data series. For a detailed explanation of the standard deviations presented in these tables, refer to Part 3 of this supplementary material. In the case of laminar burning velocity measurements (Table A4), the measurement methods are also shown (flame cone method (FCM), heat flux method (HFM), outwardly propagating flame method (OPF)).

Table A1. Shock tube ignition delay time (ST-IDT) measurements: experimental details for each XML data file and standard deviations for each data series.

| Reference | XML ID | T / K | p / atm | φ | Fuel | Oxidizer | Dopant | Diluent | Dp. | St. dev. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chen (2021) 2 | x10100003 | 1499–1939 | 1.11–1.23 | 1.00 | NH₃/H₂ | O₂ | – | Ar | 8 | 0.126 |
| Chen (2021) 2 | x10100004 | 1394–1956 | 9.25–11.36 | 1.00 | NH₃/H₂ | O₂ | – | Ar | 12 | 0.242 |
| Chen (2021) 2 | x10100005 | 1199–1825 | 1.17–1.21 | 1.00 | NH₃/H₂ | O₂ | – | Ar | 7 | 0.146 |
| Chen (2021) 2 | x10100006 | 1206–1939 | 9.32–12.60 | 1.00 | NH₃/H₂ | O₂ | – | Ar | 9 | 0.196 |
| Chen (2021) 2 | x10100007 | 1022–1600 | 1.09–1.26 | 1.00 | NH₃/H₂ | O₂ | – | Ar | 5 | 0.153 |
| Chen (2021) 2 | x10100008 | 1052–1296 | 9.23–9.96 | 1.00 | NH₃/H₂ | O₂ | – | Ar | 7 | 0.237 |
| Mathieu (2015) 3 | x10199009 | 1564–1894 | 27.70–29.70 | 0.50 | NH₃ | O₂ | – | Ar | 8 | 0.091 |
| Mathieu (2015) 3 | x10199010 | 1927–2479 | 1.29–1.58 | 0.50 | NH₃ | O₂ | – | Ar | 11 | 0.067 |
| Mathieu (2015) 3 | x10199011 | 1626–2015 | 10.40–11.40 | 0.50 | NH₃ | O₂ | – | Ar | 9 | 0.093 |
| Mathieu (2015) 3 | x10199012 | 1586–1926 | 27.70–29.60 | 1.00 | NH₃ | O₂ | – | Ar | 9 | 0.079 |
| Mathieu (2015) 3 | x10199013 | 1987–2489 | 1.29–1.51 | 1.00 | NH₃ | O₂ | – | Ar | 10 | 0.063 |
| Mathieu (2015) 3 | x10199014 | 1663–2079 | 10.60–11.20 | 1.00 | NH₃ | O₂ | – | Ar | 9 | 0.062 |
| Mathieu (2015) 3 | x10199015 | 1566–1866 | 28.00–29.70 | 1.00 | NH₃ | O₂ | – | Ar | 7 | 0.121 |
| Mathieu (2015) 3 | x10199016 | 1827–2454 | 1.25–1.52 | 1.00 | NH₃ | O₂ | – | Ar | 9 | 0.060 |
| Mathieu (2015) 3 | x10199017 | 1618–2085 | 10.10–10.90 | 1.00 | NH₃ | O₂ | – | Ar | 7 | 0.083 |
| Mathieu (2015) 3 | x10199018 | 1584–1906 | 27.50–30.40 | 2.00 | NH₃ | O₂ | – | Ar | 7 | 0.112 |
| Mathieu (2015) 3 | x10199019 | 1992–2404 | 1.35–1.49 | 2.00 | NH₃ | O₂ | – | Ar | 10 | 0.058 |
| Mathieu (2015) 3 | x10199020 | 1651–2038 | 10.10–11.30 | 2.00 | NH₃ | O₂ | – | Ar | 8 | 0.076 |
| Shu (2019) 4 | x10199029 | 1213–1492 | 19.05–19.74 | 0.50 | NH₃ | O₂ | – | N₂ | 5 | 0.155 |
| Shu (2019) 4 | x10199030 | 1181–1524 | 37.50–41.65 | 0.50 | NH₃ | O₂ | – | N₂ | 5 | 0.152 |
| Shu (2019) 4 | x10199031 | 1264–1525 | 19.25–20.33 | 1.00 | NH₃ | O₂ | – | N₂ | 5 | 0.108 |
| Shu (2019) 4 | x10199032 | 1280–1581 | 37.01–41.65 | 1.00 | NH₃ | O₂ | – | N₂ | 5 | 0.125 |
| Shu (2019) 4 | x10199033 | 1183–1543 | 15.89–19.74 | 1.99 | NH₃ | O₂ | – | N₂ | 5 | 0.175 |
| Shu (2019) 4 | x10199034 | 1195–1554 | 37.11–41.35 | 1.99 | NH₃ | O₂ | – | N₂ | 5 | 0.132 |
| Chen (2021) 2 | x10199035 | 1613–1899 | 1.01–1.31 | 1.00 | NH₃ | O₂ | – | Ar | 11 | 0.118 |
| Chen (2021) 2 | x10199036 | 1441–1787 | 9.80–10.53 | 1.00 | NH₃ | O₂ | – | Ar | 7 | 0.205 |

Table A2. Concentration profile measurements in jet stirred reactors (JSRs): experimental details for each XML data file and standard deviations for each data series.

| Reference | XML ID | T / K | p / atm | φ | Fuel | Oxidizer | Dopant | Diluent | Meas. | Dp. | St. dev. / ppm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Zhang (2021) 5 | x00100000 | 900–1277 | 1.00 | 0.15 | NH₃/H₂ | O₂ | – | N₂ | H₂O | 10 | 5–280 |
|  |  |  |  |  |  |  |  |  | NH₃ | 10 | 5–137 |
|  |  |  |  |  |  |  |  |  | NO | 10 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 10 | 5 |
| Zhang (2021) 5 | x00100001 | 900–1220 | 1.00 | 0.16 | NH₃/H₂ | O₂ | – | N₂ | H₂O | 9 | 5–165 |
|  |  |  |  |  |  |  |  |  | NH₃ | 9 | 5–69 |
|  |  |  |  |  |  |  |  |  | NO | 9 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 9 | 5 |
| Zhang (2021) 5 | x00100002 | 800–1170 | 1.00 | 0.18 | NH₃/H₂ | O₂ | – | N₂ | H₂O | 10 | 5–122 |
|  |  |  |  |  |  |  |  |  | NH₃ | 10 | 5–134 |
|  |  |  |  |  |  |  |  |  | NO | 10 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 10 | 5 |
| Zhang (2021) 5 | x00100003 | 800–1060 | 1.00 | 0.20 | NH₃/H₂ | O₂ | – | N₂ | H₂O | 9 | 5–223 |
|  |  |  |  |  |  |  |  |  | NH₃ | 9 | 5–33 |
|  |  |  |  |  |  |  |  |  | NO | 9 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 9 | 5 |
| Zhang (2021) 5 | x00100004 | 900–1281 | 1.00 | 0.62 | NH₃/H₂ | O₂ | – | N₂ | H₂O | 10 | 5–162 |
|  |  |  |  |  |  |  |  |  | NH₃ | 10 | 5–93 |
|  |  |  |  |  |  |  |  |  | NO | 10 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 10 | 5 |
| Zhang (2021) 5 | x00100005 | 900–1220 | 1.00 | 0.66 | NH₃/H₂ | O₂ | – | N₂ | H₂O | 8 | 5–322 |
|  |  |  |  |  |  |  |  |  | NH₃ | 8 | 5–66 |
|  |  |  |  |  |  |  |  |  | NO | 8 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 8 | 5 |
| Zhang (2021) 5 | x00100006 | 800–1120 | 1.00 | 0.71 | NH₃/H₂ | O₂ | – | N₂ | H₂O | 9 | 5–233 |
|  |  |  |  |  |  |  |  |  | NH₃ | 9 | 5–52 |
|  |  |  |  |  |  |  |  |  | NO | 9 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 9 | 5 |
| Zhang (2021) 5 | x00100007 | 800–1060 | 1.00 | 0.79 | NH₃/H₂ | O₂ | – | N₂ | H₂O | 9 | 5–154 |
|  |  |  |  |  |  |  |  |  | NH₃ | 9 | 5–28 |
|  |  |  |  |  |  |  |  |  | NO | 9 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 9 | 5 |
| Osipova (2022) 6 | x00100008 | 800–1300 | 1.00 | 1.00 | NH₃/H₂ | O₂ | – | Ar | H₂ | 18 | 8–39 |
|  |  |  |  |  |  |  |  |  | NH₃ | 18 | 6–136 |
|  |  |  |  |  |  |  |  |  | H₂O | 18 | 5–622 |
|  |  |  |  |  |  |  |  |  | N₂ | 18 | 5–41 |
|  |  |  |  |  |  |  |  |  | O₂ | 18 | 5–73 |
| Osipova (2022) 6 | x00100009 | 800–1300 | 1.00 | 0.60 | NH₃/H₂ | O₂ | – | Ar | H₂ | 18 | 5–27 |
|  |  |  |  |  |  |  |  |  | NH₃ | 18 | 5–110 |
|  |  |  |  |  |  |  |  |  | H₂O | 18 | 5–251 |
|  |  |  |  |  |  |  |  |  | N₂ | 18 | 5–69 |
|  |  |  |  |  |  |  |  |  | O₂ | 18 | 29–85 |
| Osipova (2022) 6 | x00100010 | 800–1300 | 1.00 | 1.50 | NH₃/H₂ | O₂ | – | Ar | H₂ | 18 | 19–32 |
|  |  |  |  |  |  |  |  |  | NH₃ | 18 | 22–112 |
|  |  |  |  |  |  |  |  |  | N₂ | 18 | 5–30 |
|  |  |  |  |  |  |  |  |  | O₂ | 18 | 5–48 |
|  |  |  |  |  |  |  |  |  | H₂O | 17 | 5–204 |
| Stagni (2020) 7 | x00199000 | 500–1200 | 1.05 | 0.02 | NH₃ | O₂ | – | He | NH₃ | 12 | 14–26 |
|  |  |  |  |  |  |  |  |  | NO | 7 | 5 |
|  |  |  |  |  |  |  |  |  | NO₂ | 7 | 5 |
| Stagni (2020) 7 | x00199001 | 500–1200 | 1.05 | 0.01 | NH₃ | O₂ | – | He | NH₃ | 8 | 6–25 |
|  |  |  |  |  |  |  |  |  | NO | 8 | 5 |
|  |  |  |  |  |  |  |  |  | NO₂ | 8 | 5 |
| Dagaut (2022) 8 | x00199003 | 1101–1452 | 1.00 | 0.06 | NH₃ | O₂ | – | N₂ | NH₃ | 15 | 5–62 |
|  |  |  |  |  |  |  |  |  | H₂O | 15 | 5–114 |
|  |  |  |  |  |  |  |  |  | NO₂ | 15 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 15 | 5–17 |
|  |  |  |  |  |  |  |  |  | N₂O | 15 | 5 |
| Dagaut (2022) 8 | x00199004 | 1253–1450 | 1.00 | 0.30 | NH₃ | O₂ | – | N₂ | NH₃ | 9 | 12–270 |
|  |  |  |  |  |  |  |  |  | H₂O | 9 | 5–90 |
|  |  |  |  |  |  |  |  |  | NO | 9 | 5 |
| Dagaut (2022) 8 | x00199005 | 1250–1452 | 1.00 | 0.60 | NH₃ | O₂ | – | N₂ | NH₃ | 9 | 17–249 |
|  |  |  |  |  |  |  |  |  | H₂O | 9 | 5–79 |
|  |  |  |  |  |  |  |  |  | NO | 9 | 5 |
| Dagaut (2022) 8 | x00199006 | 1304–1450 | 1.00 | 1.20 | NH₃ | O₂ | – | N₂ | NH₃ | 7 | 40–147 |
|  |  |  |  |  |  |  |  |  | H₂O | 7 | 5–80 |
|  |  |  |  |  |  |  |  |  | NO | 7 | 5 |
| Manna (2020) 9 | x00199007 | 953–1293 | 1.20 | 1.00 | NH₃ | O₂ | – | Ar | H₂ | 12 | 69–524 |
|  |  |  |  |  |  |  |  |  | O₂ | 12 | 43–1562 |
|  |  |  |  |  |  |  |  |  | N₂ | 12 | 5–14367 |
|  |  |  |  |  |  |  |  |  | NO | 12 | 5–5 |
| Manna (2020) 9 | x00199008 | 953–1313 | 1.20 | 1.00 | NH₃ | O₂ | H₂O | Ar | H₂ | 13 | 84–557 |
|  |  |  |  |  |  |  |  |  | O₂ | 13 | 43–1630 |
|  |  |  |  |  |  |  |  |  | N₂ | 13 | 5–11287 |
|  |  |  |  |  |  |  |  |  | NO | 13 | 5–6 |
| Sabia (2020) 10 | x00199011 | 966–1288 | 1.20 | 0.80 | NH₃ | O₂ | – | N₂ | H₂ | 30 | 9–146 |
|  |  |  |  |  |  |  |  |  | O₂ | 30 | 466–1664 |
|  |  |  |  |  |  |  |  |  | NO | 30 | 5 |
| Sabia (2020) 10 | x00199012 | 965–1293 | 1.20 | 1.00 | NH₃ | O₂ | – | N₂ | H₂ | 26 | 15–220 |
|  |  |  |  |  |  |  |  |  | O₂ | 26 | 351–13537 |
|  |  |  |  |  |  |  |  |  | NO | 26 | 5 |
| Sabia (2020) 10 | x00199013 | 1012–1293 | 1.20 | 1.20 | NH₃ | O₂ | – | N₂ | H₂ | 34 | 39–780 |
|  |  |  |  |  |  |  |  |  | O₂ | 34 | 119–2998 |
|  |  |  |  |  |  |  |  |  | NO | 34 | 5 |
| Sabia (2020) 10 | x00199014 | 1100 | 1.20 | 0.60–1.40 | NH₃ | O₂ | – | N₂ | H₂ | 26 | 43–68 |
|  |  |  |  |  |  |  |  |  | O₂ | 26 | 1163–1882 |
|  |  |  |  |  |  |  |  |  | NO | 26 | 5 |
| Sabia (2020) 10 | x00199015 | 1225 | 1.20 | 0.60–1.40 | NH₃ | O₂ | – | N₂ | H₂ | 26 | 22–61 |
|  |  |  |  |  |  |  |  |  | O₂ | 26 | 1537–3124 |
|  |  |  |  |  |  |  |  |  | NO | 26 | 5 |
| Sabia (2020) 10 | x00199016 | 1270 | 1.20 | 0.60–1.40 | NH₃ | O₂ | – | N₂ | H₂ | 18 | 12–1291 |
|  |  |  |  |  |  |  |  |  | O₂ | 18 | 335–3471 |
|  |  |  |  |  |  |  |  |  | NO | 18 | 5 |
| Sabia (2020) 10 | x00199017 | 923–1283 | 1.20 | 1.00 | NH₃ | O₂ | H₂O | N₂ | H₂ | 26 | 5–297 |
|  |  |  |  |  |  |  |  |  | O₂ | 26 | 95–3709 |
|  |  |  |  |  |  |  |  |  | NO | 26 | 5 |
| Zhang (2021) 5 | x00199018 | 900–1277 | 1.00 | 0.15 | NH₃ | O₂ | – | N₂ | H₂O | 10 | 5–149 |
|  |  |  |  |  |  |  |  |  | NH₃ | 10 | 27–106 |
|  |  |  |  |  |  |  |  |  | NO | 10 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 10 | 5 |
| Zhang (2021) 5 | x00199019 | 900–1277 | 1.00 | 0.60 | NH₃ | O₂ | – | N₂ | H₂O | 10 | 5–173 |
|  |  |  |  |  |  |  |  |  | NH₃ | 10 | 11–105 |
|  |  |  |  |  |  |  |  |  | NO | 10 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 10 | 5 |
| Osipova (2022) 6 | x00199020 | 900–1300 | 1.00 | 1.00 | NH₃ | O₂ | – | Ar | NH₃ | 24 | 30–179 |
|  |  |  |  |  |  |  |  |  | O₂ | 24 | 12–54 |
|  |  |  |  |  |  |  |  |  | H₂O | 22 | 5–174 |
|  |  |  |  |  |  |  |  |  | N₂ | 22 | 5–144 |
| Ding (2021) 11 | x00101000 | 1300 | 1.00 | 0.57 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | O₂ | 3 | 102–122 |
|  |  |  |  |  |  |  |  |  | CO | 3 | 5–31 |
|  |  |  |  |  |  |  |  |  | H₂ | 3 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 3 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 3 | 5 |
| Ding (2021) 11 | x00101001 | 1300 | 1.00 | 1.15 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | O₂ | 3 | 5–20 |
|  |  |  |  |  |  |  |  |  | CO | 3 | 15–63 |
|  |  |  |  |  |  |  |  |  | H₂ | 3 | 5–10 |
|  |  |  |  |  |  |  |  |  | N₂O | 3 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 3 | 5 |
| Ding (2021) 11 | x00101002 | 900–1300 | 1.00 | 0.57 | NH₃/H₂/CO | O₂ | – | N₂ | O₂ | 6 | 230–409 |
|  |  |  |  |  |  |  |  |  | CO | 6 | 5–104 |
|  |  |  |  |  |  |  |  |  | H₂ | 6 | 5–38 |
|  |  |  |  |  |  |  |  |  | N₂O | 6 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 6 | 5–27 |
| Ding (2021) 11 | x00101003 | 900–1350 | 1.00 | 0.57 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | O₂ | 7 | 315–624 |
|  |  |  |  |  |  |  |  |  | CO | 7 | 67–624 |
|  |  |  |  |  |  |  |  |  | H₂ | 7 | 5–58 |
|  |  |  |  |  |  |  |  |  | N₂O | 7 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 7 | 5–8 |
| Ding (2021) 11 | x00101004 | 900–1300 | 1.00 | 1.15 | NH₃/H₂/CO | O₂ | – | N₂ | O₂ | 6 | 5–56 |
|  |  |  |  |  |  |  |  |  | CO | 6 | 154–1520 |
|  |  |  |  |  |  |  |  |  | H₂ | 6 | 22–329 |
|  |  |  |  |  |  |  |  |  | NO | 6 | 5–15 |
|  |  |  |  |  |  |  |  |  | N₂O | 5 | 5 |
| Ding (2021) 11 | x00101005 | 900–1350 | 1.00 | 1.15 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | O₂ | 7 | 102–1465 |
|  |  |  |  |  |  |  |  |  | CO | 7 | 520–1233 |
|  |  |  |  |  |  |  |  |  | H₂ | 7 | 5–76 |
|  |  |  |  |  |  |  |  |  | N₂O | 7 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 7 | 5–13 |
| Ding (2021) 11 | x00101006 | 1300 | 1.00 | 0.40–1.37 | NH₃/H₂/CO | O₂ | – | N₂ | O₂ | 7 | 5–1333 |
|  |  |  |  |  |  |  |  |  | CO | 7 | 11–213 |
|  |  |  |  |  |  |  |  |  | H₂ | 7 | 5–30 |
|  |  |  |  |  |  |  |  |  | N₂O | 7 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 7 | 5–21 |
| Ding (2021) 11 | x00101007 | 1300 | 1.00 | 0.38–1.51 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | O₂ | 7 | 8–1605 |
|  |  |  |  |  |  |  |  |  | CO | 7 | 54–393 |
|  |  |  |  |  |  |  |  |  | H₂ | 7 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 7 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 7 | 9–18 |

Table A3. Concentration profile measurements in flow reactors (FRs): experimental details for each XML data file and standard deviations for each dataset.

| Reference | XML ID | T / K | p / atm | φ | Fuel | Oxidizer | Dopant | Diluent | Meas. | Dp. | St. dev. / ppm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wargadalam (2000) 12 | x30100001 | 873–1273 | 1.00 | 0.00 | NH₃/H₂ | O₂ | – | N₂ | NH₃ | 9 | 5–26 |
|  |  |  |  |  |  |  |  |  | NO | 9 | 5–12 |
| Hulgaard (1993) 13 | x30199001 | 938–1373 | 1.04 | 0.02 | NH₃ | O₂ | H₂O | – | NO | 11 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 10 | 5 |
| Hulgaard (1993) 13 | x30199003 | 974–1369 | 1.04 | 0.02 | NH₃ | O₂ | – | N₂ | N₂O | 10 | 5 |
|  |  |  |  |  |  |  |  |  | NH₃ | 7 | 5–142 |
|  |  |  |  |  |  |  |  |  | NO | 12 | 5 |
| Wargadalam (2000) 12 | x30199006 | 873–1274 | 1.00 | 0.00 | NH₃ | O₂ | – | N₂ | NH₃ | 9 | 5–12 |
|  |  |  |  |  |  |  |  |  | NO | 9 | 5 |
| Song (2016) 14 | x30199007 | 453–925 | 29.61 | 0.62 | NH₃ | O₂ | – | N₂ | O₂ | 7 | 28–29 |
|  |  |  |  |  |  |  |  |  | NH₃ | 7 | 37–39 |
| Song (2016) 14 | x30199009 | 451–902 | 98.69 | 0.62 | NH₃ | O₂ | – | N₂ | O₂ | 6 | 25–27 |
|  |  |  |  |  |  |  |  |  | NH₃ | 6 | 34–36 |
| Song (2016) 14 | x30199011 | 453–900 | 29.61 | 0.01 | NH₃ | O₂ | – | N₂ | O₂ | 6 | 5 |
|  |  |  |  |  |  |  |  |  | NH₃ | 6 | 24–109 |
| Song (2016) 14 | x30199013 | 453–925 | 29.61 | 0.01 | NH₃ | O₂ | – | N₂ | O₂ | 5 | 5 |
|  |  |  |  |  |  |  |  |  | NH₃ | 5 | 28–42 |
| Song (2016) 14 | x30199015 | 692–902 | 98.69 | 0.01 | NH₃ | O₂ | – | N₂ | N₂O | 4 | 5–10 |
|  |  |  |  |  |  |  |  |  | NH₃ | 9 | 5–39 |
| Song (2016) 14 | x30199017 | 692–902 | 98.69 | 0.01 | NH₃ | O₂ | – | N₂ | N₂O | 9 | 5–9 |
| Caton (1995) 15 | x30199018 | 798–1200 | 1.01 | 0.00 | NH₃ | O₂ | – | N₂ | NO | 15 | 5–6 |
|  |  |  |  |  |  |  |  |  | N₂O | 15 | 5 |
|  |  |  |  |  |  |  |  |  | NH₃ | 15 | 5–50 |
| Dean (1982) 16 | x30199019 | 1279 | 1.18 | 0.02 | NH₃ | O₂ | – | He | NH₃ | 4 | 17–49 |
|  |  |  |  |  |  |  |  |  | NO | 4 | 5 |
| Dean (1982) 16 | x30199020 | 1279 | 1.18 | 0.02 | NH₃ | O₂ | H₂O | – | NH₃ | 8 | 42–87 |
|  |  |  |  |  |  |  |  |  | NO | 8 | 5 |
| Stagni (2020) 7 | x30199021 | 1073–1973 | 1.25 | 0.38 | NH₃ | O₂ | – | He | NH₃ | 19 | 5–55 |
|  |  |  |  |  |  |  |  |  | O₂ | 19 | 76–134 |
|  |  |  |  |  |  |  |  |  | NO | 19 | 5–5 |
|  |  |  |  |  |  |  |  |  | N₂ | 19 | 5–27 |
|  |  |  |  |  |  |  |  |  | H₂O | 19 | 5–113 |
|  |  |  |  |  |  |  |  |  | H₂ | 19 | 5 |
| Abián (2021) 17 | x30199023 | 875–1475 | 1.00 | 2.44 | NH₃ | O₂ | – | N₂ | NH₃ | 16 | 27–49 |
|  |  |  |  |  |  |  |  |  | O₂ | 16 | 5–11 |
|  |  |  |  |  |  |  |  |  | H₂ | 16 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 16 | 5 |
|  |  |  |  |  |  |  |  |  | NO₂ | 16 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 16 | 5 |
| Abián (2021) 17 | x30199024 | 875–1425 | 1.00 | 1.34 | NH₃ | O₂ | – | N₂ | NH₃ | 14 | 24–52 |
|  |  |  |  |  |  |  |  |  | O₂ | 14 | 11–30 |
|  |  |  |  |  |  |  |  |  | H₂ | 14 | 5–5 |
|  |  |  |  |  |  |  |  |  | NO | 14 | 5 |
|  |  |  |  |  |  |  |  |  | NO₂ | 14 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 14 | 5 |
| Abián (2021) 17 | x30199025 | 875–1425 | 1.00 | 0.94 | NH₃ | O₂ | – | N₂ | NH₃ | 13 | 5–49 |
|  |  |  |  |  |  |  |  |  | O₂ | 13 | 5–35 |
|  |  |  |  |  |  |  |  |  | H₂ | 13 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 13 | 5 |
|  |  |  |  |  |  |  |  |  | NO₂ | 13 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 13 | 5 |
| Abián (2021) 17 | x30199026 | 875–1425 | 1.00 | 0.87 | NH₃ | O₂ | – | N₂ | NH₃ | 14 | 5–46 |
|  |  |  |  |  |  |  |  |  | O₂ | 14 | 5–66 |
|  |  |  |  |  |  |  |  |  | H₂ | 14 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 14 | 5 |
|  |  |  |  |  |  |  |  |  | NO₂ | 14 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 14 | 5 |
| Abián (2021) 17 | x30199027 | 875–1425 | 1.00 | 0.47 | NH₃ | O₂ | – | N₂ | NH₃ | 15 | 5–50 |
|  |  |  |  |  |  |  |  |  | O₂ | 15 | 70–129 |
|  |  |  |  |  |  |  |  |  | H₂ | 15 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 15 | 5–44 |
|  |  |  |  |  |  |  |  |  | NO₂ | 15 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 15 | 5 |
| Abián (2021) 17 | x30199028 | 875–1425 | 1.00 | 0.19 | NH₃ | O₂ | – | N₂ | NH₃ | 13 | 5–44 |
|  |  |  |  |  |  |  |  |  | O₂ | 13 | 99–124 |
|  |  |  |  |  |  |  |  |  | H₂ | 13 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 13 | 5–28 |
|  |  |  |  |  |  |  |  |  | NO₂ | 13 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 13 | 5 |
| Abián (2021) 17 | x30199029 | 1025–1425 | 1.00 | 0.09 | NH₃ | O₂ | – | N₂ | NH₃ | 15 | 5–54 |
|  |  |  |  |  |  |  |  |  | O₂ | 15 | 192–220 |
|  |  |  |  |  |  |  |  |  | H₂ | 15 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 15 | 5–23 |
|  |  |  |  |  |  |  |  |  | NO₂ | 15 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 15 | 5 |
| Abián (2021) 17 | x30199030 | 875–1425 | 1.00 | 0.05 | NH₃ | O₂ | – | N₂ | NH₃ | 15 | 5–52 |
|  |  |  |  |  |  |  |  |  | O₂ | 15 | 418–438 |
|  |  |  |  |  |  |  |  |  | H₂ | 15 | 5 |
|  |  |  |  |  |  |  |  |  | NO | 15 | 5 |
|  |  |  |  |  |  |  |  |  | NO₂ | 15 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 15 | 5 |
| Abián (2021) 17 | x30199031 | 875–1450 | 1.00 | 0.88 | NH₃ | O₂ | H₂O | – | NH₃ | 16 | 5–48 |
|  |  |  |  |  |  |  |  |  | O₂ | 16 | 5–42 |
|  |  |  |  |  |  |  |  |  | H₂ | 16 | 5–7 |
|  |  |  |  |  |  |  |  |  | NO | 16 | 5 |
|  |  |  |  |  |  |  |  |  | NO₂ | 16 | 5 |
|  |  |  |  |  |  |  |  |  | N₂O | 16 | 5 |
| Wargadalam (2000) 12 | x30101006 | 873–1273 | 1.00 | 0.01 | NH₃/CO | O₂ | – | N₂ | NH₃ | 9 | 5–18 |
|  |  |  |  |  |  |  |  |  | NO | 9 | 5–8 |

Table A4. Laminar burning velocity (LBV) measurements: experimental details for each XML data file and standard deviations for each data series.

| Reference | XML ID | T / K | p / atm | φ | Fuel | Oxidizer | Dopant | Diluent | Dp. | Methodc | St. dev. / cm s−1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lee (2010a) 18 | x20100004 | 298 | 1.00 | 0.60 | NH₃/H₂ | O₂ | – | N₂ | 5 | OPF | 1.98–5.74 |
| Lee (2010a) 18 | x20100005 | 298 | 1.00 | 1.00 | NH₃/H₂ | O₂ | – | N₂ | 5 | OPF | 6.53–15.60 |
| Lee (2010a) 18 | x20100006 | 298 | 1.00 | 1.67 | NH₃/H₂ | O₂ | – | N₂ | 5 | OPF | 4.12–21.63 |
| Lee (2010b) 19 | x20100007 | 298 | 1.00 | 0.60 | NH₃/H₂ | O₂ | – | N₂ | 2 | OPF | 0.50–1.06 |
| Lee (2010b) 19 | x20100008 | 298 | 1.00 | 0.80 | NH₃/H₂ | O₂ | – | N₂ | 2 | OPF | 1.00–2.04 |
| Lee (2010b) 19 | x20100009 | 298 | 1.00 | 1.00 | NH₃/H₂ | O₂ | – | N₂ | 3 | OPF | 0.91–3.03 |
| Lee (2010b) 19 | x20100010 | 298 | 1.00 | 1.25 | NH₃/H₂ | O₂ | – | N₂ | 2 | OPF | 1.36–2.74 |
| Lee (2010b) 19 | x20100011 | 298 | 1.00 | 1.67 | NH₃/H₂ | O₂ | – | N₂ | 1 | OPF | 1.46 |
| Ichikawa (2015) 20 | x20100012 | 298 | 0.99 | 1.00 | NH₃/H₂ | O₂ | – | N₂ | 9 | OPF | 0.50–19.39 |
| Ichikawa (2015) 20 | x20100013 | 298 | 0.99 | 1.00 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 0.34–0.77 |
| Ichikawa (2015) 20 | x20100014 | 298 | 0.99 | 1.00 | NH₃/H₂ | O₂ | – | N₂ | 6 | OPF | 0.50–0.56 |
| Han (2019) 21 | x20100018 | 298 | 1.00 | 1.00 | NH₃/H₂ | O₂ | – | N₂ | 19 | HFM | 0.50 |
| Han (2019) 21 | x20100019 | 298 | 1.00 | 0.75–1.40 | NH₃/H₂ | O₂ | – | N₂ | 14 | HFM | 0.50 |
| Han (2019) 21 | x20100020 | 298 | 1.00 | 0.70–1.60 | NH₃/H₂ | O₂ | – | N₂ | 19 | HFM | 0.50 |
| Han (2019) 21 | x20100021 | 298 | 1.00 | 0.70–1.60 | NH₃/H₂ | O₂ | – | N₂ | 19 | HFM | 0.50 |
| Han (2019) 21 | x20100022 | 298 | 1.00 | 0.70–1.60 | NH₃/H₂ | O₂ | – | N₂ | 19 | HFM | 0.50 |
| S. Wang (2020) 22 | x20100023 | 298 | 0.99 | 0.60–1.60 | NH₃/H₂ | O₂ | – | N₂ | 15 | HFM | 0.61–0.76 |
| S. Wang (2020) 22 | x20100024 | 298 | 2.96 | 0.60–1.60 | NH₃/H₂ | O₂ | – | N₂ | 15 | HFM | 0.60–0.87 |
| S. Wang (2020) 22 | x20100025 | 298 | 0.99 | 0.70–1.50 | NH₃/H₂ | O₂ | – | N₂ | 12 | HFM | 0.50–0.89 |
| S. Wang (2020) 22 | x20100026 | 298 | 2.96 | 0.70–1.60 | NH₃/H₂ | O₂ | – | N₂ | 13 | HFM | 0.50 |
| S. Wang (2020) 22 | x20100027 | 298 | 4.93 | 0.70–1.50 | NH₃/H₂ | O₂ | – | N₂ | 12 | HFM | 0.50 |
| Lhuillier (2020) 23 | x20100028 | 298 | 0.99 | 0.80–1.20 | NH₃/H₂ | O₂ | – | N₂ | 5 | OPF | 0.74–0.80 |
| Lhuillier (2020) 23 | x20100029 | 323 | 0.99 | 0.80–1.30 | NH₃/H₂ | O₂ | – | N₂ | 6 | OPF | 0.70–0.80 |
| Lhuillier (2020) 23 | x20100030 | 373 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 0.85–1.38 |
| Lhuillier (2020) 23 | x20100031 | 423 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 0.91–1.39 |
| Lhuillier (2020) 23 | x20100032 | 473 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 0.71–1.31 |
| Lhuillier (2020) 23 | x20100033 | 298 | 0.99 | 0.80–1.30 | NH₃/H₂ | O₂ | – | N₂ | 6 | OPF | 0.83–0.99 |
| Lhuillier (2020) 23 | x20100034 | 323 | 0.99 | 0.80–1.30 | NH₃/H₂ | O₂ | – | N₂ | 6 | OPF | 0.53–0.75 |
| Lhuillier (2020) 23 | x20100035 | 373 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 0.97–1.13 |
| Lhuillier (2020) 23 | x20100036 | 423 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 1.38–1.87 |
| Lhuillier (2020) 23 | x20100037 | 473 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 0.76–2.30 |
| Lhuillier (2020) 23 | x20100038 | 298 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 0.66–0.84 |
| Lhuillier (2020) 23 | x20100039 | 323 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 0.86–1.05 |
| Lhuillier (2020) 23 | x20100040 | 373 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 1.22–1.44 |
| Lhuillier (2020) 23 | x20100041 | 423 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 1.37–1.66 |
| Lhuillier (2020) 23 | x20100042 | 473 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 1.18–2.22 |
| Lhuillier (2020) 23 | x20100043 | 298 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 1.57–1.71 |
| Lhuillier (2020) 23 | x20100044 | 323 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 1.15–1.47 |
| Lhuillier (2020) 23 | x20100045 | 373 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 1.44–1.74 |
| Lhuillier (2020) 23 | x20100046 | 423 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 1.65–2.54 |
| Lhuillier (2020) 23 | x20100047 | 473 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 2.32–2.71 |
| Lhuillier (2020) 23 | x20100048 | 298 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 1.55–2.09 |
| Lhuillier (2020) 23 | x20100049 | 323 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 1.67–2.28 |
| Lhuillier (2020) 23 | x20100050 | 373 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 1.45–2.51 |
| Lhuillier (2020) 23 | x20100051 | 423 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 2.33–3.24 |
| Lhuillier (2020) 23 | x20100052 | 473 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 2.57–3.64 |
| Lhuillier (2020) 23 | x20100053 | 298 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 2.03–2.63 |
| Lhuillier (2020) 23 | x20100054 | 323 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 1.64–2.02 |
| Lhuillier (2020) 23 | x20100055 | 373 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 3.30–3.59 |
| Lhuillier (2020) 23 | x20100056 | 423 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 3.05–3.42 |
| Lhuillier (2020) 23 | x20100057 | 473 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 3.43–4.80 |
| Lhuillier (2020) 23 | x20100058 | 298 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 2.94–3.66 |
| Lhuillier (2020) 23 | x20100059 | 323 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 3.25–4.35 |
| Lhuillier (2020) 23 | x20100060 | 373 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 3.65–5.51 |
| Lhuillier (2020) 23 | x20100061 | 423 | 0.99 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 5.25–6.02 |
| Lhuillier (2020) 23 | x20100062 | 473 | 0.99 | 0.80–1.30 | NH₃/H₂ | O₂ | – | N₂ | 6 | OPF | 5.74–6.95 |
| Shrestha (2021) 24 | x20100063 | 472 | 2.92 | 1.10 | NH₃/H₂ | O₂ | – | N₂ | 1 | OPF | 0.50 |
| Shrestha (2021) 24 | x20100064 | 473–476 | 2.89–2.94 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 0.88–1.13 |
| Shrestha (2021) 24 | x20100065 | 470 | 2.88 | 1.10 | NH₃/H₂ | O₂ | – | N₂ | 1 | OPF | 0.50 |
| Shrestha (2021) 24 | x20100066 | 470 | 2.90 | 1.10 | NH₃/H₂ | O₂ | – | N₂ | 1 | OPF | 0.50 |
| Shrestha (2021) 24 | x20100067 | 468–474 | 4.84–4.87 | 1.10 | NH₃/H₂ | O₂ | – | N₂ | 4 | OPF | 0.50–1.04 |
| Shrestha (2021) 24 | x20100068 | 468–475 | 6.86–6.88 | 1.10 | NH₃/H₂ | O₂ | – | N₂ | 4 | OPF | 0.50–1.53 |
| Shrestha (2021) 24 | x20100069 | 476 | 9.43 | 1.10 | NH₃/H₂ | O₂ | – | N₂ | 1 | OPF | 3.32 |
| Gotama (2022) 25 | x20100070 | 298 | 0.99 | 0.80–1.80 | NH₃/H₂ | O₂ | – | N₂ | 8 | OPF | 0.17–0.58 |
| Gotama (2022) 25 | x20100071 | 298 | 4.93 | 1.00–1.80 | NH₃/H₂ | O₂ | – | N₂ | 6 | OPF | 0.50–0.74 |
| Mei (2021) 26 | x20100072 | 298 | 1.00 | 0.70–1.40 | NH₃/H₂ | O₂ | – | N₂ | 8 | OPF | 0.56–0.79 |
| Mei (2021) 26 | x20100073 | 298 | 2.00 | 0.80–1.40 | NH₃/H₂ | O₂ | – | N₂ | 7 | OPF | 0.92–1.42 |
| Mei (2021) 26 | x20100074 | 298 | 5.00 | 1.10–1.40 | NH₃/H₂ | O₂ | – | N₂ | 4 | OPF | 0.38–0.49 |
| Mei (2021) 26 | x20100075 | 298 | 10.00 | 1.10–1.40 | NH₃/H₂ | O₂ | – | N₂ | 4 | OPF | 0.43–0.71 |
| Mei (2021) 26 | x20100076 | 298 | 1.00 | 1.00 | NH₃/H₂ | O₂ | – | N₂ | 8 | OPF | 0.50–5.53 |
| Mei (2021) 26 | x20100077 | 298 | 2.00 | 1.00 | NH₃/H₂ | O₂ | – | N₂ | 6 | OPF | 0.41–1.32 |
| Mei (2021) 26 | x20100078 | 298 | 5.00 | 1.00 | NH₃/H₂ | O₂ | – | N₂ | 3 | OPF | 0.50–0.98 |
| Wang (2021) 27 | x20100079 | 360 | 0.99 | 0.50–1.50 | NH₃/H₂ | O₂ | – | N₂ | 11 | OPF | 0.50–1.92 |
| Wang (2021) 27 | x20100080 | 360 | 0.99–4.93 | 1.00 | NH₃/H₂ | O₂ | – | N₂ | 3 | OPF | 0.50–1.53 |
| Wang (2021) 27 | x20100081 | 360 | 0.99–4.93 | 1.00 | NH₃/H₂ | O₂ | – | N₂ | 3 | OPF | 0.50–2.85 |
| Osipova (2021) 28 | x20100082 | 368 | 1.00 | 0.70–1.50 | NH₃/H₂ | O₂ | – | N₂ | 9 | FCM | 2.58–4.51 |
| Hayakawa (2015) 29 | x20199002 | 298 | 0.99 | 0.80–1.20 | NH₃ | O₂ | – | N₂ | 5 | OPF | 0.50 |
| Hayakawa (2015) 29 | x20199003 | 298 | 2.96 | 0.90–1.20 | NH₃ | O₂ | – | N₂ | 4 | OPF | 0.50 |
| Hayakawa (2015) 29 | x20199004 | 298 | 4.93 | 0.90–1.20 | NH₃ | O₂ | – | N₂ | 4 | OPF | 0.50 |
| Han (2019) 21 | x20199005 | 298 | 1.00 | 0.85–1.25 | NH₃ | O₂ | – | N₂ | 9 | HFM | 1.41 |
| Liu (2019) 30 | x20199006 | 298 | 0.50 | 0.50–1.30 | NH₃ | O₂ | – | – | 4 | OPF | 5.57–8.85 |
| Liu (2019) 30 | x20199007 | 298 | 0.70 | 0.50–1.30 | NH₃ | O₂ | – | – | 3 | OPF | 5.27–7.42 |
| Liu (2019) 30 | x20199008 | 298 | 1.00 | 0.40–2.00 | NH₃ | O₂ | – | – | 10 | OPF | 1.97–17.76 |
| Liu (2019) 30 | x20199009 | 298 | 1.40 | 0.75–1.75 | NH₃ | O₂ | – | – | 4 | OPF | 4.91–11.34 |
| Liu (2019) 30 | x20199010 | 298 | 1.60 | 0.50–1.75 | NH₃ | O₂ | – | – | 5 | OPF | 3.80–9.44 |
| Mei (2019) 31 | x20199011 | 298 | 1.00 | 0.90–1.30 | NH₃ | O₂ | – | N₂ | 5 | OPF | 0.50 |
| Mei (2019) 31 | x20199012 | 298 | 1.00 | 0.60–1.50 | NH₃ | O₂ | – | N₂ | 10 | OPF | 0.50–0.83 |
| Mei (2019) 31 | x20199013 | 298 | 2.00 | 0.60–1.50 | NH₃ | O₂ | – | N₂ | 10 | OPF | 0.40–0.52 |
| Mei (2019) 31 | x20199014 | 298 | 5.00 | 0.60–1.50 | NH₃ | O₂ | – | N₂ | 10 | OPF | 0.36–0.50 |
| Mei (2019) 31 | x20199015 | 298 | 1.00 | 0.70 | NH₃ | O₂ | – | N₂ | 5 | OPF | 0.50–2.16 |
| Mei (2019) 31 | x20199016 | 298 | 1.00 | 1.00 | NH₃ | O₂ | – | N₂ | 6 | OPF | 0.50–1.55 |
| Mei (2019) 31 | x20199017 | 298 | 1.00 | 1.50 | NH₃ | O₂ | – | N₂ | 5 | OPF | 0.43–0.62 |
| D. Wang (2020) 32 | x20199018 | 303 | 0.99 | 0.60–1.40 | NH₃ | O₂ | – | N₂ | 9 | OPF | 2.18–3.34 |
| D. Wang (2020) 32 | x20199019 | 303 | 0.99 | 0.60–1.40 | NH₃ | O₂ | – | N₂ | 9 | OPF | 2.20–3.36 |
| D. Wang (2020) 32 | x20199020 | 303 | 0.99 | 0.60–1.40 | NH₃ | O₂ | – | – | 9 | OPF | 2.15–3.52 |
| D. Wang (2020) 32 | x20199021 | 303–393 | 0.99 | 0.70 | NH₃ | O₂ | – | N₂ | 4 | OPF | 1.40–2.69 |
| D. Wang (2020) 32 | x20199022 | 303–393 | 0.99 | 0.70 | NH₃ | O₂ | – | N₂ | 4 | OPF | 2.64–4.74 |
| D. Wang (2020) 32 | x20199023 | 303–393 | 0.99 | 0.70 | NH₃ | O₂ | – | – | 4 | OPF | 3.24–4.39 |
| D. Wang (2020) 32 | x20199024 | 303–393 | 0.99 | 1.40 | NH₃ | O₂ | – | N₂ | 4 | OPF | 1.73–1.99 |
| D. Wang (2020) 32 | x20199025 | 303–393 | 0.99 | 1.40 | NH₃ | O₂ | – | N₂ | 4 | OPF | 1.78–2.42 |
| D. Wang (2020) 32 | x20199026 | 303–393 | 0.99 | 1.40 | NH₃ | O₂ | – | – | 4 | OPF | 2.00–3.00 |
| Lhuillier (2020) 23 | x20199027 | 298 | 0.99 | 0.90–1.10 | NH₃ | O₂ | – | N₂ | 3 | OPF | 0.57–0.64 |
| Lhuillier (2020) 23 | x20199028 | 323 | 0.99 | 0.90–1.20 | NH₃ | O₂ | – | N₂ | 4 | OPF | 0.50–0.63 |
| Lhuillier (2020) 23 | x20199029 | 373 | 0.99 | 0.90–1.30 | NH₃ | O₂ | – | N₂ | 5 | OPF | 0.50–0.70 |
| Lhuillier (2020) 23 | x20199030 | 423 | 0.99 | 0.80–1.30 | NH₃ | O₂ | – | N₂ | 6 | OPF | 0.75–0.91 |
| Lhuillier (2020) 23 | x20199031 | 473 | 0.99 | 0.80–1.40 | NH₃ | O₂ | – | N₂ | 7 | OPF | 0.97–1.14 |
| Shrestha (2021) 24 | x20199032 | 472–475 | 2.92–2.98 | 0.80–1.40 | NH₃ | O₂ | – | N₂ | 7 | OPF | 0.85–0.93 |
| Shrestha (2021) 24 | x20199033 | 298 | 0.99 | 0.90–1.10 | NH₃ | O₂ | – | N₂ | 3 | OPF | 0.50 |
| Shrestha (2021) 24 | x20199034 | 298 | 0.99 | 0.90–1.20 | NH₃ | O₂ | – | N₂ | 4 | OPF | 0.50 |
| Shrestha (2021) 24 | x20199035 | 298 | 0.99 | 0.80–1.30 | NH₃ | O₂ | – | N₂ | 6 | OPF | 0.59–2.15 |
| Shrestha (2021) 24 | x20199036 | 298 | 0.99 | 0.80–1.30 | NH₃ | O₂ | – | N₂ | 6 | OPF | 0.50–1.19 |
| Shrestha (2021) 24 | x20199037 | 298 | 0.99 | 0.80–1.30 | NH₃ | O₂ | – | N₂ | 6 | OPF | 0.50–0.95 |
| Shrestha (2021) 24 | x20199038 | 323 | 0.99 | 0.90–1.20 | NH₃ | O₂ | – | N₂ | 4 | OPF | 0.50–0.60 |
| Shrestha (2021) 24 | x20199039 | 323 | 0.99 | 0.90–1.20 | NH₃ | O₂ | – | N₂ | 4 | OPF | 0.50–0.61 |
| Shrestha (2021) 24 | x20199040 | 323 | 0.99 | 0.80–1.30 | NH₃ | O₂ | – | N₂ | 6 | OPF | 0.75–1.24 |
| Shrestha (2021) 24 | x20199041 | 323 | 0.99 | 0.80–1.30 | NH₃ | O₂ | – | N₂ | 6 | OPF | 0.70–1.40 |
| Shrestha (2021) 24 | x20199042 | 323 | 0.99 | 0.80–1.30 | NH₃ | O₂ | – | N₂ | 6 | OPF | 0.72–1.39 |
| Shrestha (2021) 24 | x20199043 | 373 | 0.99 | 1.00–1.30 | NH₃ | O₂ | – | N₂ | 4 | OPF | 0.50–2.48 |
| Shrestha (2021) 24 | x20199044 | 373 | 0.99 | 0.80–1.30 | NH₃ | O₂ | – | N₂ | 6 | OPF | 0.69–3.56 |
| Shrestha (2021) 24 | x20199045 | 373 | 0.99 | 0.80–1.30 | NH₃ | O₂ | – | N₂ | 6 | OPF | 0.90–1.67 |
| Shrestha (2021) 24 | x20199046 | 373 | 0.99 | 0.80–1.30 | NH₃ | O₂ | – | N₂ | 6 | OPF | 0.63–1.43 |
| Shrestha (2021) 24 | x20199047 | 373 | 0.99 | 0.80–1.30 | NH₃ | O₂ | – | N₂ | 6 | OPF | 0.93–1.52 |
| Shrestha (2021) 24 | x20199048 | 298 | 0.99 | 0.90–1.10 | NH₃ | O₂ | – | He | 3 | OPF | 0.64–1.51 |
| Shrestha (2021) 24 | x20199049 | 298 | 0.99 | 0.90–1.20 | NH₃ | O₂ | – | He | 4 | OPF | 0.83–0.98 |
| Takeishi (2015) 33 | x20199050 | 298 | 1.00 | 0.90–1.40 | NH₃ | O₂ | – | N₂ | 8 | FCM | 0.73–1.42 |
| Takeishi (2015) 33 | x20199051 | 298 | 1.00 | 0.80–1.40 | NH₃ | O₂ | – | N₂ | 9 | FCM | 1.06–1.71 |
| Takeishi (2015) 33 | x20199052 | 298 | 1.00 | 0.80–1.40 | NH₃ | O₂ | – | N₂ | 11 | FCM | 1.47–2.31 |
| N. Wang (2021) 27 | x20199053 | 360 | 0.99 | 0.70–1.40 | NH₃ | O₂ | – | N₂ | 8 | OPF | 0.50 |
| N. Wang (2021) 27 | x20199054 | 360 | 0.99–4.93 | 1.00 | NH₃ | O₂ | – | N₂ | 3 | OPF | 0.50 |
| Han (2019) 21 | x20101000 | 298 | 1.00 | 1.00 | NH₃/CO | O₂ | – | N₂ | 44 | HFM | 0.50 |
| Han (2019) 21 | x20101001 | 298 | 1.00 | 0.80–1.45 | NH₃/CO | O₂ | – | N₂ | 14 | HFM | 0.50 |
| Han (2019) 21 | x20101002 | 298 | 1.00 | 0.70–1.60 | NH₃/CO | O₂ | – | N₂ | 19 | HFM | 0.50 |
| Han (2019) 21 | x20101003 | 298 | 1.00 | 0.70–1.60 | NH₃/CO | O₂ | – | N₂ | 19 | HFM | 0.50 |
| Han (2019) 21 | x20101004 | 298 | 1.00 | 0.70–1.70 | NH₃/CO | O₂ | – | N₂ | 21 | HFM | 0.50–0.52 |
| Han (2020) 34 | x20101005 | 298 | 1.00 | 1.00 | NH₃/H₂/CO | O₂ | – | N₂ | 42 | HFM | 0.52–0.58 |
| Han (2020) 34 | x20101006 | 298 | 1.00 | 0.85–1.40 | NH₃/H₂/CO | O₂ | – | N₂ | 12 | HFM | 0.50 |
| Han (2020) 34 | x20101007 | 298 | 1.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 19 | HFM | 0.50 |
| Han (2020) 34 | x20101008 | 298 | 1.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 19 | HFM | 0.50 |
| Han (2020) 34 | x20101009 | 298 | 1.00 | 1.00 | NH₃/H₂/CO | O₂ | – | N₂ | 29 | HFM | 0.50–0.51 |
| Han (2020) 34 | x20101010 | 298 | 1.00 | 0.80–1.40 | NH₃/H₂/CO | O₂ | – | N₂ | 13 | HFM | 0.50 |
| Han (2020) 34 | x20101011 | 298 | 1.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 19 | HFM | 0.50 |
| Han (2020) 34 | x20101012 | 298 | 1.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 19 | HFM | 0.50 |
| Mei (2020) | x20101013 | 298 | 1.00 | 1.00 | NH₃/H₂/CO | O₂ | – | N₂ | 6 | OPF | 0.50–4.85 |
| Mei (2020) 35 | x20101014 | 298 | 1.00 | 1.00 | NH₃/H₂/CO | O₂ | – | N₂ | 5 | OPF | 0.50–4.28 |
| Mei (2020) 35 | x20101015 | 298 | 1.00 | 1.00 | NH₃/H₂/CO | O₂ | – | N₂ | 5 | OPF | 0.72–8.33 |
| Mei (2020) 35 | x20101016 | 298 | 1.00 | 0.70–1.50 | NH₃/H₂/CO | O₂ | – | N₂ | 9 | OPF | 0.47–0.60 |
| Mei (2020) 35 | x20101017 | 298 | 2.00 | 0.70–1.50 | NH₃/H₂/CO | O₂ | – | N₂ | 9 | OPF | 0.38–0.50 |
| Mei (2020) 35 | x20101018 | 298 | 5.00 | 0.90–1.50 | NH₃/H₂/CO | O₂ | – | N₂ | 7 | OPF | 0.36–0.50 |
| Mei (2020) 35 | x20101019 | 298 | 10.00 | 1.10–1.40 | NH₃/H₂/CO | O₂ | – | N₂ | 4 | OPF | 0.37–0.50 |
| Mei (2020) 35 | x20101020 | 298 | 1.00 | 0.70–1.50 | NH₃/H₂/CO | O₂ | – | N₂ | 9 | OPF | 0.60–0.97 |
| Mei (2020) 35 | x20101021 | 298 | 2.00 | 0.70–1.50 | NH₃/H₂/CO | O₂ | – | N₂ | 9 | OPF | 0.48–0.81 |
| Mei (2020) 35 | x20101022 | 298 | 5.00 | 1.10–1.50 | NH₃/H₂/CO | O₂ | – | N₂ | 5 | OPF | 0.48–0.68 |
| Mei (2020) 35 | x20101023 | 298 | 10.00 | 1.10–1.40 | NH₃/H₂/CO | O₂ | – | N₂ | 4 | OPF | 0.50–0.76 |
| Mei (2020) 35 | x20101024 | 298 | 1.00 | 0.70–1.50 | NH₃/H₂/CO | O₂ | – | N₂ | 9 | OPF | 0.74–1.35 |
| Mei (2020) 35 | x20101025 | 298 | 2.00 | 0.70–1.50 | NH₃/H₂/CO | O₂ | – | N₂ | 9 | OPF | 0.64–1.22 |
| Mei (2020) 35 | x20101026 | 298 | 5.00 | 1.10–1.50 | NH₃/H₂/CO | O₂ | – | N₂ | 5 | OPF | 0.40–0.52 |
| Mei (2020) 35 | x20101027 | 298 | 10.00 | 1.10–1.50 | NH₃/H₂/CO | O₂ | – | N₂ | 5 | OPF | 0.47–0.61 |
| S. Wang (2020) 22 | x20101028 | 298 | 1.00 | 1.00 | NH₃/H₂/CO | O₂ | – | N₂ | 17 | HFM | 0.50–0.72 |
| S. Wang (2020) 22 | x20101029 | 298 | 3.00 | 1.00 | NH₃/H₂/CO | O₂ | – | N₂ | 14 | HFM | 0.50–0.90 |
| S. Wang (2020) 22 | x20101030 | 298 | 5.00 | 1.00 | NH₃/H₂/CO | O₂ | – | N₂ | 13 | HFM | 0.50 |
| S. Wang (2020) 22 | x20101031 | 298 | 1.00 | 1.00 | NH₃/CO | O₂ | – | N₂ | 21 | HFM | 0.50–0.53 |
| S. Wang (2020) 22 | x20101032 | 298 | 3.00 | 1.00 | NH₃/CO | O₂ | – | N₂ | 21 | HFM | 0.50–0.89 |
| S. Wang (2020) 22 | x20101033 | 298 | 5.00 | 1.00 | NH₃/CO | O₂ | – | N₂ | 18 | HFM | 0.50 |
| S. Wang (2020) 22 | x20101034 | 298 | 1.00 | 0.80–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 11 | HFM | 0.50–0.65 |
| S. Wang (2020) 22 | x20101035 | 298 | 3.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 12 | HFM | 0.50–0.55 |
| S. Wang (2020) 22 | x20101036 | 298 | 5.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 10 | HFM | 0.50 |
| S. Wang (2020) 22 | x20101037 | 298 | 1.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 12 | HFM | 0.50–0.51 |
| S. Wang (2020) | x20101038 | 298 | 3.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 11 | HFM | 0.50 |
| S. Wang (2020) 22 | x20101039 | 298 | 5.00 | 0.80–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 9 | HFM | 0.51–0.63 |
| S. Wang (2020) 22 | x20101040 | 298 | 1.00 | 0.80–1.60 | NH₃/CO | O₂ | – | N₂ | 11 | HFM | 0.50–0.64 |
| S. Wang (2020) 22 | x20101041 | 298 | 3.00 | 0.70–1.60 | NH₃/CO | O₂ | – | N₂ | 12 | HFM | 0.50 |
| S. Wang (2020) 22 | x20101042 | 298 | 5.00 | 0.70–1.40 | NH₃/CO | O₂ | – | N₂ | 10 | HFM | 0.50 |
| S. Wang (2020) 22 | x20101043 | 298 | 1.00 | 0.80–1.60 | NH₃/CO | O₂ | – | N₂ | 11 | HFM | 0.50 |
| S. Wang (2020) 22 | x20101044 | 298 | 3.00 | 0.80–1.40 | NH₃/CO | O₂ | – | N₂ | 9 | HFM | 0.50 |
| S. Wang (2020) 22 | x20101045 | 298 | 5.00 | 0.80–1.30 | NH₃/CO | O₂ | – | N₂ | 8 | HFM | 0.50 |
| Zhou (2021) 36 | x20101046 | 297–298 | 1.00 | 0.70–1.41 | NH₃/H₂/CO | O₂ | – | N₂ | 8 | OPF | 0.47–1.04 |
| Zhou (2021) 36 | x20101047 | 322–323 | 1.00 | 0.71–1.41 | NH₃/H₂/CO | O₂ | – | N₂ | 8 | OPF | 0.49–1.33 |
| Zhou (2021) 36 | x20101048 | 372–373 | 1.00 | 0.70–1.42 | NH₃/H₂/CO | O₂ | – | N₂ | 8 | OPF | 0.56–1.34 |
| Zhou (2021) 36 | x20101049 | 422–423 | 1.00 | 0.71–1.41 | NH₃/H₂/CO | O₂ | – | N₂ | 8 | OPF | 0.87–1.34 |
| Zhou (2021) 36 | x20101050 | 298 | 1.00 | 0.71–1.41 | NH₃/H₂/CO | O₂ | – | N₂ | 8 | OPF | 0.62–1.48 |
| Zhou (2021) 36 | x20101051 | 323 | 1.00 | 0.70–1.42 | NH₃/H₂/CO | O₂ | – | N₂ | 8 | OPF | 0.47–1.37 |
| Zhou (2021) 36 | x20101052 | 372–373 | 1.00 | 0.71–1.41 | NH₃/H₂/CO | O₂ | – | N₂ | 8 | OPF | 0.72–1.26 |
| Zhou (2021) 36 | x20101053 | 422–423 | 1.00 | 0.70–1.40 | NH₃/H₂/CO | O₂ | – | N₂ | 8 | OPF | 1.05–1.87 |
| Zhou (2021) 36 | x20101054 | 297–298 | 1.00 | 0.81–1.42 | NH₃/H₂/CO | O₂ | – | N₂ | 7 | OPF | 0.73–1.40 |
| Zhou (2021) 36 | x20101055 | 323 | 1.00 | 0.81–1.41 | NH₃/H₂/CO | O₂ | – | N₂ | 7 | OPF | 1.31–2.09 |
| Zhou (2021) 36 | x20101056 | 372–373 | 1.00 | 0.81–1.41 | NH₃/H₂/CO | O₂ | – | N₂ | 7 | OPF | 1.09–1.91 |
| Zhou (2021) 36 | x20101057 | 422–423 | 1.00 | 0.91–1.41 | NH₃/H₂/CO | O₂ | – | N₂ | 6 | OPF | 1.34–2.20 |
| Zhou (2021) 36 | x20101058 | 298 | 1.00 | 0.98–1.02 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 11 | OPF | 0.50–1.76 |
| Zhou (2021) 36 | x20101059 | 298 | 1.00 | 0.80–1.30 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 6 | OPF | 0.50–0.61 |
| Zhou (2021) 36 | x20101060 | 323 | 1.00 | 0.80–1.31 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 6 | OPF | 0.50–0.78 |
| Zhou (2021) 36 | x20101061 | 372–373 | 1.00 | 0.80–1.31 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 6 | OPF | 0.80–1.09 |
| Zhou (2021) 36 | x20101062 | 422–423 | 1.00 | 0.81–1.32 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 6 | OPF | 0.73–0.98 |
| Zhou (2021) 36 | x20101063 | 297–298 | 1.00 | 0.81–1.31 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 6 | OPF | 0.39–0.81 |
| Zhou (2021) 36 | x20101064 | 323 | 1.00 | 0.80–1.31 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 6 | OPF | 0.54–0.89 |
| Zhou (2021) 36 | x20101065 | 372–373 | 1.00 | 0.80–1.31 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 6 | OPF | 0.76–1.05 |
| Zhou (2021) 36 | x20101066 | 422–423 | 1.00 | 0.81–1.32 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 6 | OPF | 0.50–1.00 |
| Zhou (2021) 36 | x20101067 | 298 | 1.00 | 0.79–1.32 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 6 | OPF | 0.44–1.15 |
| Zhou (2021) 36 | x20101068 | 322–323 | 1.00 | 0.71–1.41 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 8 | OPF | 0.59–1.01 |
| Zhou (2021) 36 | x20101069 | 372–373 | 1.00 | 0.70–1.42 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 8 | OPF | 0.68–1.05 |
| Zhou (2021) 36 | x20101070 | 422–423 | 1.00 | 0.70–1.40 | NH₃/H₂/CO | O₂ | – | CO₂/N₂ | 8 | OPF | 0.74–1.01 |
| Yin (2021) 37 | x20101071 | 298 | 1.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 10 | OPF | 0.91–1.50 |
| Yin (2021) 37 | x20101072 | 373 | 1.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 10 | OPF | 0.59–1.52 |
| Yin (2021) 37 | x20101073 | 443 | 1.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 10 | OPF | 0.99–2.24 |
| Yin (2021) 37 | x20101074 | 298 | 2.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 10 | OPF | 0.39–0.86 |
| Yin (2021) 37 | x20101075 | 298 | 3.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 10 | OPF | 0.31–0.62 |
| Yin (2021) 37 | x20101076 | 298 | 1.00 | 1.00–1.20 | NH₃/H₂/CO | O₂ | – | N₂ | 3 | OPF | 0.50 |
| Yin (2021) 37 | x20101077 | 298 | 1.00 | 0.70–1.50 | NH₃/H₂/CO | O₂ | – | N₂ | 9 | OPF | 0.32–0.87 |
| Yin (2021) 37 | x20101078 | 298 | 1.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 10 | OPF | 0.58–1.43 |
| Yin (2021) 37 | x20101079 | 298 | 1.00 | 0.70–1.60 | NH₃/H₂/CO | O₂ | – | N₂ | 10 | OPF | 0.54–1.22 |

# Overview of excluded experimental data

In Part 1 of this supplementary material, details of experiments that were utilized in this work are summarized. There are other papers in the literature which contain experiments with our target systems, but for some reasons they were excluded from our investigations. These experiments are discussed in this part.

In the case of ST-IDT experiments, these excluded papers were published in the 1960s–‍1980s38-49, and the measured ignition delay times are not reproducible due to the lack of unambiguous experimental details and/or IDT definitions. Moreover, as stated in the work of Chen et al. 2: “The ignition delay times of ammonia have been reported since the 1960s–80s, however, those early shock tube studies present the undesirable repeatability and low experimental accuracy. Especially, the adsorption of ammonia on the contact surface of the stainless steel equipment was not considered which lead to large experimental uncertainty.”

Mathieu et al. 50 investigated NH3/syngas mixtures in their ST-IDT studies, but in those cases, NH3 was present only as a trace species in the gas mixtures, it was not a main component of the fuel. Since this kind of system is not the focus of this study, these results are excluded from the investigations presented here.

In the works of Manna et al. 9 and Sabia et al. 10, flow reactor experiments on NH3 oxidation and pyrolysis can also be found in addition to the JSR experiments. However, according to the authors, these measurements “cannot be straightforwardly used for modeling activities. For this reason, simulations were performed considering the data from the JSR”. Therefore, these flow reactor measurements were not used in this study either. In the works of Nakamura et al. 51,52, a special kind of flow reactor (micro flow reactor) was used, the modeling of which is more complicated and not possible using our programs.

There are also LBV measurements whose results were not utilized in the present study. In the work of Kumar and Meyer53, the inlet conditions of the experiments are not published unambiguously; therefore, these results cannot be reproduced accurately by simulations. Moreover, they used the Bunsen burner method for the flame speed determination which suffers from stretch effects and heat loss21, and the authors applied correction only for the heat loss. In the works of Li et al. 54 Takizawa et al. 55, Jabbour et al. 56, Pfahl et al. 57, Ronney58, and Zakaznov et al. 59, the flames were affected by stretch effects, and no stretch correction was applied to calculate the real LBVs2,19,22,29. Davis et al. 60 investigated the oxidation of ammonia in a laboratory-scale version of an industrial combustor. They measured the maximal LBV of the investigated gas mixtures, so these experimental results were also not used in this study.

# Simulation of ST-IDT experiments

In the work of Shu et al. 4, the pressure profiles were measured during the experiments with reactive mixtures and significant pressure rises were observed before the ignitions especially in the case of longer ignition delay times (i.e. at lower temperatures). The pressure rises probably occurred because i) N2 was used as the diluent gas instead of a monatomic gas (e.g. Ar) and ii) the dilution levels of the initial gas mixtures were not high (xN2 = 50–70%). The experimental pressure profiles were converted to volume profiles by the authors using the thermochemical data of the model of Mathieu et al. 3 the assumption of isentropic compression61,62. The volume profiles were obtained from B. Shu through personal communication, and these volume profiles were used as inputs in our simulations. The reported temperature and pressure values behind the reflected shock waves were used as initial conditions for the simulations and the volume profiles accounted for the pressure rises. According to Shu et al., it was not possible to perform measurements with non-reactive mixtures that could represent the dynamic effects of the reactive mixtures because i) it was not possible to reproduce the conditions of the reactive measurements using the non-reactive mixtures and ii) the reactive mixtures had too strong dynamic effects so replacing the O2 by N2 in the non-reactive experiments and using those data as simulation inputs would introduce large uncertainty in the simulation results.

Mathieu et al. 3 reported that the pre-ignition pressure rise rates were less than 2% / ms in their ST-IDT experiments, which would result in temperature rises of a maximum of 10 K. According to the authors, these temperature rises do not have a noticeable impact on the results, and therefore the pressure rise was not considered in these simulations. These observations correspond with the fact that they used gas mixtures highly diluted in Ar (xAr > 98%). Also, they reported an uncertainty of 10% for the measured ignition delay times which included the uncertainty of temperature due to the pressure rise. Chen et al. 2 also experienced pressure rise rates of less than 2% / ms in their ST-IDT measurements. Since they also used Ar as the diluent gas (xAr = 92–‍93% in the initial mixtures), we neglected the pressure rise in our simulations.

# Estimation of the standard deviation of the experimental data

For the quantitative comparison of mechanism performance, the following error function was used:

| E=1Nf=1Nfs=1Nfs1Nfsdd=1NfsdYfsdsim-YfsdexpσYfsdexp2 | (1) |
| --- | --- |

where

| Yfsd= yfsdif σyfsdexp≈constantlnyfsdif σlnyfsdexp≈constant | (2) |
| --- | --- |

and

- N: number of data series in the data collection

- Nf : number of datasets (XML data files) in the data collection

- index f goes through all datasets (XML data files) in the data collection

- Nfs: number of data series in the f-th dataset (XML data file)

- index s goes through all data series in the f-th dataset (XML data file)

- Nfsd: number of data points in the s-th data series in the f-th dataset (XML data file)

- index d goes through all data points in the s-th data series in the f-th dataset (XML data file)

- yfsdexp and σyfsdexp are the experimentally measured value and standard deviation of the d-th data point in the s-th data series in the f-th dataset, respectively,

- yfsdsim is the simulation result for d-th data point in the s-th data series in the f-th dataset.

To estimate the standard deviation of a data point, both the experimental standard deviation (σexp,fsd) of the data point and the statistical scatter of the s-th data series in the f-th dataset (σfit,fs) are considered as follows:

| σYfsdexp=σfit,fs2+σexp,fsd2 . | (3) |
| --- | --- |

The σfit,fs term stems from the statistical scatter of the data points around a trendline fitted to the data series. To estimate σfit,fs, the Minimal Spline Fit program of Dr. Tibor Nagy63,64 was used. In the webpage of the ReSpecTh database, currently “version January 5, 2020” of the program is available along with its user manual.

According to equation (3), the experimental error reported by the experimenters is also considered to calculate the overall σ for a data point. Also, the type of the experimental error (absolute or relative) is important because it determines which model (constant absolute or relative standard deviation) is used when the statistical scatter of the data series is estimated. The features of the experimental errors are discussed in this section for each measured quantity. In most publications where experimental errors are reported, they refer to either the ±2σ uncertainty limits or the 95% confidence intervals which are approximately equal to ±2σ in the case of normal distribution. Therefore, unless otherwise stated, published experimental uncertainties are considered as 2σ.

## Ignition delay time measurements

In Table A5, the reported experimental errors are collected from the investigated ST-IDT measurement papers. Ignition delay time measurements typically have a constant relative error65,66; therefore, the constant relative error model was used to estimate the statistical scatter. The overall (relative) standard deviations (σ) of the transformed ST-IDT measurement data series are summarized in Table A1, together with some selected experimental details.

Table A5. Experimental errors reported in the shock tube ignition delay time measurement papers.

| Reference | Apparatus | Type of experimental error | Error value (2σ) |
| --- | --- | --- | --- |
| Mathieu (2015) 3 | ST | relative | 10% |
| Shu (2019) 4 | ST | relative | 20% |
| Chen (2021) 2 | ST | relative | 20% |

## Concentration measurements

Table A6. Reported experimental error types of the investigated species concentrations measurements in jet stirred reactors (JSR) and flow reactors (FR).

| Reference | Measured species | Measurement methoda | Type of experimental error |
| --- | --- | --- | --- |
| JSR |  |  |  |
| Dagaut (2019) 8 | NH3, NO, NO2, N2O, H2O: | FT-IR | relative |
| Stagni (2020) 7 | NH3: NO, NO2: | cw-CRDS chemiluminescence anal. | relative |
| Sabia (2020) 10 | NO: H2, O2, N2: | NDIR gas sensor GC | relative (with absolute minimum limit for NO) |
| Manna (2020) 9 | NO: H2, O2, N2: | NDIR gas sensor GC | relative (with absolute minimum limit for NO) |
| Zhang (2021) 5 | NH3, NO, N2O, H2O: | FT-IR | relative |
| Osipova (2022) 6 | NH3, H2O, H2, N2, O2: | MS | no error is reported |
| Ding (2021) 11 | NO, CO: N2O: H2, O2: | FT-IR + NDIR gas sensor FT-IR NDIR gas sensor | relative |
| FR |  |  |  |
| Dean (1982) 16 | NH3, NO: | chemiluminescence anal. | no error is reported |
| Hulgaard (1993) 13 | NH3: NO: N2O: | spectrophotometry NDUV gas sensor NDIR gas sensor | relative with absolute minimum limits |
| Caton (1995) 15 | NH3, NO, N2O: | FT-IR | no error is reported |
| Wargadalam (2000) 12 | NH3, NO: | FT-IR | relative |
| Song (2016) 14 | NH3, N2O: O2: | NDIR gas sensor GC | relative |
| Stagni (2020) 7 | NH3, H2O, N2, O2: NO: H2: | MS chemiluminescence anal. GC | relative |
| Abián (2021) 17 | NH3, N2O, H2, O2: NO2: NO: | GC chemiluminescence anal. NDIR gas sensor + chemiluminescence anal. | relative with absolute minimum limits |

a: 	FT-IR = Fourier Transform Infrared Spectroscopy,
cw-CRDS = continuous-wave cavity ring-down spectroscopy (it is based on IR absorption),
chemiluminescence anal. = chemiluminescence NOx analyzer,
NDIR = non-dispersive ultraviolet,
NDUV = non-dispersive infrared,
GC = gas chromatography,
MS = mass spectrometry.

According to the work of Olm et al. 65, results of concentration measurements should be characterized by absolute errors. Therefore, even if the overall standard deviations (σ) were obtained using the relative error model, the relative σ values were then converted to absolute ones and those values were used in the error function calculation (Yfsd = yfsd in equation (1)).

Concentrations of chemical species may be measured by various means, of which optical spectroscopy (usually in the infrared (IR) range) and gas chromatography (GC) were most frequently applied in the investigated experiments. Besides those techniques, mass spectrometry was also used in some cases (Table A6).

Table A7. Minimum overall relative and absolute errors defined for each species for concentration measurements in jet stirred reactors and flow reactors.

| Species | σ min rel | σ min abs ppm |
| --- | --- | --- |
| NH3 | 0.05 (5%) | 5 |
| NO | 0.025 (2.5%) | 5 |
| NO2 | 0.05 (5%) | 5 |
| N2O | 0.05 (5%) | 5 |
| H2O | 0.05 (5%) | 5 |
| H2 | 0.025 (2.5%) | 5 |
| N2 | 0.025 (2.5%) | 5 |
| O2 | 0.025 (2.5%) | 5 |
| CO | 0.025 (2.5%) | 5 |

In these measurements, if the experimental uncertainty is published, it is always a relative error (Table A6). Therefore, constant relative error was assumed during the estimation of σfit, that is, polynomials or splines were fitted by Minimal Spline Fit to the logarithm of the data.

The authors often publish a minimum absolute limit for the experimental error. It means that the absolute value of the error cannot be smaller than that limit. If it is smaller, the data point has an absolute error, which is important for small concentration values. To avoid using unrealistically small standard deviations, a minimum relative (σ min rel) and absolute standard deviation (σ min abs) were defined for each species based on previous experience from the literature (Table A7). These values helped to estimate the overall standard deviation (σ) of the data series realistically.

For each data series, the relative statistical scatter of the data series was estimated using the program Minimal Spline Fit. If the experimental standard deviation was reported in the publication, then equation (3) was used to estimate the overall standard deviation (σ) of the data series. In some cases, both the statistical scatter and the experimental error were very small for a data point, and the resulting overall σ was smaller than σ min rel. If this happened, σ min rel was taken as σ to avoid using unrealistically small standard deviations.

If the experimental standard deviation was not reported for the measurement, σ min rel replaced σexp in equation (3) as follows:

| σ=σfit2+σminrel2, | (4) |
| --- | --- |

which is inherently not smaller than σ min rel.

In a few cases, it was not possible to fit an appropriate model function to the data series, for instance, because the number of data points in the data series was too small. In these cases, if the experimental error was provided, σ min rel was assigned to σfit in equation (3) as follows:

| σ=σminrel2+σexp2. | (5) |
| --- | --- |

If it was not provided, σ min rel was assigned to both as σfit and σexp, so:

| σ=2σminrel. | (6) |
| --- | --- |

Let us denote the resulting relative σ value obtained using either equation (3), (4), (5), or (6) by σrel. For each data point, the absolute standard deviation was computed as (concentration value · σrel). If this value was not smaller than σ min abs of the respective species, then σrel was accepted and relative error was considered for that data point. If it was smaller than σ min abs, σ min abs was assigned to that data point and absolute error was considered.

From the previous arguments, it follows that data points within one data series may have different absolute standard deviations in the case of concentration measurements. The overall absolute standard deviation values (or value ranges) are listed for each concentration data series in Table A2 for JSR measurements and Table A3 for FR measurements, together with some selected experimental details of the corresponding experiments. These values (or value ranges) were obtained as follows. For all data points, the absolute values of the standard deviations were computed (these were used in the evaluation of the error function). If these values were the same for each data point within a data series, that single value was assigned to the data series and is written in the appropriate table. If these values were not uniform for each point of a data series, then their ranges (minimum value–maximum value) are shown in the appropriate table.

Note that when the measured concentration of a species was zero, it was omitted from the relative error estimation, and σ min abs of the respective species was assigned to that data point as an absolute error.

Finally, another concern arises when estimating the standard deviation of the logarithmically transformed concentration data, which is also discussed in the work of Kawka et al. 67. It is not a good strategy to include concentration values that are relatively small within a data series in the fitting processs using Minimal Spline Fit, because on the logarithmic scale, these data points would artificially be overweighted and bias the regression. Sometimes, if these points were included in the data series in the model fitting process, no appropriate model function could be found, but when these points were excluded, the other points could be described well by a model function. Therefore, concentration values that were more than ten times smaller than the largest value of the data series were sometimes excluded from the fitting process. A σfit value – that was calculated based on the relative standard deviation estimated using all the other data points in the data series – was assigned to these points.

## Laminar burning velocity measurements

The very recent work of Zhang et al. 68 analyzes the experimental errors of laminar burning velocities measured by various experimental techniques including those that are used in the papers investigated in this study. The results of Zhang et al. 68 helped to estimate the standard deviations of laminar burning velocity measurements. According to the works of Olm et al. 65 and Zhang et al. 68, results of laminar burning velocity measurements should be characterized by absolute errors. Therefore, even if the overall standard deviations (σ) were estimated using the relative error model, the relative σ values were then converted to absolute ones and those values were used in the error function calculation (Yfsd = yfsd in equation (1)). The experimental publications can be divided into three cases based on how the experimental uncertainties are published. Table A8 shows which investigated publications belong to each case. The details of the three cases are discussed below.

- Case 1:

A constant relative experimental error is published for all data points within a data series. In these cases, constant relative error was assumed for the data series, so the statistical standard deviation was estimated using the constant relative error model. Consequently, all points within a data series had the same relative error, while the absolute error values lay within a range.

- Case 2:

Pointwise absolute or relative experimental errors are published, that is, a different experimental error value (absolute or relative) is assigned to each data point within a data series. In these cases, it is difficult to decide which error model should be applied to the data series. Zhang et al. 68 reported that the measurements performed with the HFM method typically have a constant absolute error; therefore, the constant absolute error model was used to estimate the statistical scatter in these cases. In contrast, data series obtained by the OPF or FCM methods have a constant relative error68, so the constant relative error model was used for these data. There were two exceptions to these rules. In the OPF measurements of Lhuillier et al. 23 and Shrestha et al. 24, relative errors are published for each data point. In these cases, the constant absolute error assumption seemed more appropriate based on the investigation of the error values.

- Case 3:

In some publications, the experimental errors are not reported. Zhang et al. 68 determined the typical standard deviations of laminar burning velocity measurements, they are

- 1 cm / s absolute standard deviation for HFM measurements,

- 5% relative standard deviation for OPF measurements, and

- 6% relative standard deviation for FCM measurements.

These σexp values were assigned to experiments of this case and the corresponding error types were used to estimate the statistical scatter.

Table A8. Categorization of laminar burning velocity measurement papers based on how experimental uncertainties are published (FCM =  flame cone method, OPF = outwardly propagating flame method, HFM = heat flux burner method).

| Cases | Publication |
| --- | --- |
| Case 1: | Constant relative experimental error for each data series - FCM measurements: Osipova (2021) 28 - OPF measurements: Lee (2010a) 18, Lee (2010b) 19 |
| Case 2: | Pointwise errors - Absolute pointwise errors: - OPF measurements: Hayakawa (2015) 29, Ichikawa (2015) 20, Mei (2019) 31, D. Wang (2020) 32, Mei (2021) 26, N. Wang (2021) 27, Gotama (2022) 25, Mei (2020) 35, Zhou (2021) 36, Yin (2021) 37 - HFM measurements: Han (2019) 21, Han (2020) 34, S. Wang (2020) 22 - Relative pointwise errors: - OPF measurements: Lhuillier (2020) 23, Shrestha (2021) 24 |
| Case 3: | No experimental error is published - FCM measurements: Takeishi (2015) 33 - OPF measurements: Liu (2019) 30 |

As can be seen in Table A8, only FCM and OPF measurements belonged to this case. For these methods, the constant relative error model was utilized, and therefore different absolute error value was assigned to each point within a data series.

Note that in a few cases, it was not possible to fit an appropriate model function to the data series, for instance, because the number of data points in the data series was too small. In these cases, the measurement-specific typical error values (see in the description of Case 3) were assigned to the σfit of the data points of the data series.

Similar to concentration measurements (see Section 3.2), both the experimental (σexp) and the statistical (σfit) standard deviations were very small for some data points. If this happened, the overall standard deviation (σ) of the data point was also very small. This may lead to a very high error function value for that data point, which would result in an artificially very high overall error function value. To avoid this, global minimum overall standard deviations were introduced68: a minimum relative standard deviation of σ min rel = 1%, and minimum absolute standard deviation of σ min abs = 0.5 cm / s. The goal was to keep both the absolute value and the minimum value of the standard deviation of each point above these limiting values. This was achieved as described below.

For data points with a relative error, it was checked whether the corresponding σ was smaller than σ min rel. If it was not, the absolute value of the standard deviation was calculated as (LBV value · σ). If this value was greater than σ min abs, the overall relative σ was accepted, but if not, σ min abs was assigned to the data point. If σ was smaller σ min rel, the same procedure was carried out but using σ min rel as the relative σ.

For data points with an absolute error, first, the corresponding (σ / LBV value) relative standard deviation was calculated. If it was not smaller than σ min rel, it was checked whether the absolute σ was smaller than σ min abs. If it was not, the absolute σ was accepted, but if it was, σ min abs was assigned to the data point. If the (σ / LBV value) was smaller than σ min rel, σ was not accepted, and the (LBV value · σ min rel) absolute standard deviation was calculated. If it was not smaller than σ min abs, it was accepted as the absolute standard deviation, but if it was, σ min abs was assigned to the data point.

In Table A4, overall absolute standard deviation values (or value ranges) are listed for the investigated data series.

# Investigated reaction mechanisms

Some selected features of the mechanisms utilized in this study are summarized in Table A9. Some of the mechanisms aim to describe more complex combustion systems than NH3/syngas oxidation (Table A10). Hence, the following two reductions of the original mechanisms were applied before the simulations to decrease the computational effort.

1) For NH3/H2 combustion simulations: All species (and the corresponding reactions) were removed from the mechanisms that contain an atom other than H, O, N, and noble gases. The numbers of species and reactions that remained in the models after this kind of reduction are listed in the “H/O/N part” column in Table A9.

2) For NH3/syngas combustion simulations: In addition to the species and reactions included in the H/O/N part, those species (and the corresponding reactions) were also included in the mechanisms that contain a maximum of one carbon atom (such as CO, CO2, or HCN). The numbers of species and reactions that remained in the models after this reduction are listed in the “H/O/N + C1 part” column in Table A9.

Mechanisms that do not contain a wet CO submechanism cannot be used for the simulation of NH3/syngas combustion experiments. Table A9 shows that out of the 18 investigated mechanisms five can be used only for the simulation of NH3/H2 experiments.

In a few experiments, He was used as the diluent gas instead of N2 or Ar. Since He is not applied frequently as a diluent, not all mechanisms contain the species He. These four mechanisms cannot be used for the simulations of these experiments.

In the case of some utilized ST-IDT measurements2-4, the ignition delay time was defined based on the concentration profile of the excited OH radical (OH*). However, Table A9 shows that six mechanisms do not contain the OH* species and its reactions. To overcome this issue, the OH* submechanism of the syngas combustion mechanism of Kéromnès et al. 69 was added to these mechanisms, and ST-IDT simulations were carried out using these modified models.

Table A9. Detailed reaction mechanisms investigated in this study and their selected features. Mechanisms are listed in chronological order (the years in the mechanism IDs refer to the publication year of the corresponding mechanism).

| Mechanism ID | H/O/N part |  |  |  | H/O/N + C1 part |  |  |  | Wet CO submechanism |  | He species |  | OH* submechanism |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Species |  | Reactions |  | Species |  | Reactions |  |  |  |  |  |  |  |
| Tian-200970, a |  | 31 |  | 191 |  | 60 |  | 508 |  | ✔️ |  |  |  |  |
| Mathieu-20153 |  | 33 |  | 159 |  | 52 |  | 273 |  | ✔️ |  | ✔️ |  | ✔️ |
| GDFKin-201671 |  | 22 |  | 124 |  | 45 |  | 350 |  | ✔️ |  |  |  |  |
| Nakamura-201752 |  | 34 |  | 229 |  | – |  | – |  |  |  | ✔️ |  | ✔️ |
| SanDiego-201872, b |  | 21 |  | 64 |  | 33 |  | 129 |  | ✔️ |  | ✔️ |  |  |
| Otomo-201873 |  | 32 |  | 213 |  | – |  | – |  |  |  | ✔️ |  | ✔️ |
| Glarborg-201874 |  | 34 |  | 221 |  | 80 |  | 674 |  | ✔️ |  | ✔️ |  | ✔️ |
| Okafor-201875 |  | 25 |  | 101 |  | 46 |  | 262 |  | ✔️ |  |  |  |  |
| POLIMI-20207,76 |  | 31 |  | 203 |  | – |  | – |  |  |  | ✔️ |  |  |
| NUIG-202077, c |  | 37 |  | 231 |  | 43 |  | 253 |  | ✔️ |  | ✔️ |  | ✔️ |
| Han-202034 |  | 32 |  | 163 |  | 35 |  | 177 |  | ✔️ |  | ✔️ |  | ✔️ |
| Mei-202035 |  | 34 |  | 237 |  | 38 |  | 255 |  | ✔️ |  | ✔️ |  | ✔️ |
| Konnov-202178 |  | 36 |  | 295 |  | 80 |  | 858 |  | ✔️ |  | ✔️ |  | ✔️ |
| KAUST-20215 |  | 34 |  | 262 |  | – |  | – |  |  |  | ✔️ |  | ✔️ |
| Shrestha-202124 |  | 33 |  | 262 |  | 63 |  | 609 |  | ✔️ |  | ✔️ |  | ✔️ |
| Mei-202126 |  | 35 |  | 239 |  | 39 |  | 257 |  | ✔️ |  | ✔️ |  | ✔️ |
| Zhou-202136 |  | 32 |  | 187 |  | 34 |  | 197 |  | ✔️ |  |  |  | ✔️ |
| Gotama-202225 |  | 32 |  | 165 |  | – |  | – |  |  |  | ✔️ |  | ✔️ |

a: Thermochemical data of the carbon atom (C) was missing from the Tian-2009 model; therefore, for NH3/syngas combustion simulations, they were taken from the Glarborg-2018 model.

b: The mechanism is composed of the 2016/12/14 version of hydrocarbon oxidation and the 2018/07/23 version of the nitrogen chemistry sub-mechanism by the Combustion Research Group at the University of California at San Diego.

c: The detailed kinetic mechanism of the authors was used because the indirect experiments investigated in this study were not used for constructing the skeletal mechanism published in the same paper.

Note that the list of mechanisms in Table A9 is not exhaustive; there are many other mechanisms available in the literature14,31,79-88 that can be used to simulate NH3/syngas combustion systems. However, they were not selected among the investigated models because a) they have gone through one or more updates since their publication, b) their focus is a system different from our target systems, and/or c) they are reduced mechanisms developed for a specific system. The recent, optimized NH3 combustion model of Bertolino et al. 89 was not inves35tigated in this study, but we are planning to include it in our future studies on ammonia combustion.

The literature of detailed reaction mechanisms in ammonia combustion has been extensively reviewed, and we assume that the review is comprehensive till January, 2022.

Table A10. Main target systems of the detailed kinetic mechanisms investigated in this study.

| Mechanism ID | Main target system | Validation targets |
| --- | --- | --- |
| Tian-200970 | Speciation in premixed NH3/CH4/O2/Ar BSFs at low p. | Speciation in premixed NH3/CH4/O2/Ar BSFs at low p. |
| Mathieu-20153 | Oxidation of NH3 at high T and related NOx chemistry (mostly ST-IDT experiments). | ST-IDT experiments on NH3 pyrolysis and oxidation, H2 and syngas oxidation by N2O or O2, and HCN oxidation. ST concentration profiles on HCN oxidation. JSR concentration measurements on H2 oxidation by N2O and doped with NO. |
| GDFKin-201671 | Prompt NO formation in premixed alkane BSFs at low p. | HCN oxidation in BSF and JSR, HNCO oxidation in FR. Prompt NO formation in premixed alkane (mostly CH4 but also C2H2, C2H6, C3H8, C4H10) BSFs at low p. |
| Nakamura-201752 | Speciation in NH3/air flames at atmospheric p and low T. | Speciation in NH3/air flames at atmospheric p and low T. ST-IDT data on H2 oxidation with O2/N2O/NO2, N2H4 pyrolysis, and NH3 oxidation. LBV data on NH3 oxidation. |
| SanDiego-201872 | NOx emission in hydrocarbon flames. | Not published. |
| Otomo-201873 | LBVs and IDTs during NH3 and NH3/H2 oxidation. | ST-IDT, LBV, and BSF speciation data on NH3 oxidation. LBV data on NH3/H2 oxidation. |
| Glarborg-201874 | Comprehensive N-chemistry model during the combustion of light hydrocarbons and fuel-N species (HCN, NH3, HNCO). | Extensively validated against various types of experimental data related to N-chemistry including BSF speciation, ST-IDT, and FR concentration measurements on NH3 oxidation. |
| Okafor-201875 | LBVs of CH4/NH3/air flames. | LBVs of CH4/NH3/air flames. |
| POLIMI-20207,76 | Comprehensive NH3 pyrolysis and oxidation model. | Extensively validated against experimental data on NH3 oxidation and pyrolysis using ST-IDT, RCM-IDT, LBV, FR and JSR concentration, and BSF speciation measurements. |
| NUIG-202077 | Comprehensive syngas/NOx combustion model. | ST-IDT data on H2 oxidation by O2/N2O/NO2 and NH3 oxidation. LBV data H2 oxidation by N2O, JSR data on H2 oxidation. FR experiments on thermal NO formation. |
| Han-202034 | LBVs of premixed NH3/syngas/air flames. | LBV data on NH3, NH3/H2, NH3/CO, and NH3/syngas oxidation. ST-IDT data on NH3 oxidation, syngas oxidation with NH3 impurities, and H2 oxidation by O2/N2O/NO2. Speciation measurements on NH3/NO/Ar in BSFs, JSR concentration measurements on H2 oxidation by O2/N2O doped with NO. |
| Mei-202035 | LBVs of premixed NH3/syngas/air flames up to 10 atm. | LBV data on NH3, NH3/H2, and NH3/syngas oxidation. ST-IDT data on syngas oxidation with NH3 impurities. |
| Konnov-202178 | LBVs of premixed CH4/HCOOH/air flames. | LBV data on CH4/HCOOH oxidation. The H2/O2 and NH3/NOx submechanisms in Konnov-2021 are the same as those in the model published by Han et al. 90, which was validated against LBV data on NH3 by O2/N2O and NH3/H2 oxidation, and ST-IDT data on H2 oxidation by N2O. |
| KAUST-20215 | Comprehensive NH3 and NH3/H2 oxidation model. | Extensively validated against ST-IDT, RCM-IDT, LBV, FR and JSR concentration, and BSF speciation measurements including a large number of experiments on NH3 and NH3/H2 oxidation. See the Supplementary Material of the paper for details. |
| Shrestha-202124 | LBVs of NH3/O2-enriched air and NH3/H2/air flames up to 10 bar and 473 K. | Extensively validated against ST-IDT, RCM-IDT, LBV, FR and JSR concentration, and BSF speciation measurements including data on NH3 and NH3/H2 oxidation. See the Supplementary Material of the paper for details. |
| Mei-202126 | LBVs of premixed NH3/H2/N2/air flames up to 10 atm. | LBV data on NH3 and NH3/H2 oxidation, and BSF speciation data on NH3/H2 oxidation and H2 oxidation doped with NH3. |
| Zhou-202136 | LBVs of premixed NH3/(bio)syngas/air flames up to 423 K. | LBV data on syngas, NH3, NH3/H2, and NH3/syngas oxidation. ST-IDT data on syngas oxidation doped with NH3. |
| Gotama-202225 | LBVs of premixed NH3/H2/air flames. | LBVs of premixed NH3/H2/air flames. |

# Quantitative mechanism comparison

## NH3/H2 fuel mixtures

In the main paper, those experiments in which He is used as the diluent gas were not included in the investigations because four of the investigated mechanisms do not contain that species. The Eaverageall values for the other 14 mechanisms including the experiments containing He can be found in Table A11. The number of all investigated data points is 3770 which are contained in 350 data series in 207 datasets. Including also these data in the mechanism comparison does not change the order of the models significantly; the three best-performing mechanisms (in order) are still POLIMI-2020, Han-2020, and KAUST-2021 according to the Eaverageall values.

Table A11. Averaged error function values for 14 of the 18 investigated mechanisms in the case of NH3/H2 fuel mixtures including experiments containing He as diluent. The other four mechanisms were not included in this investigation because they do not contain the species He. See Sections 4.1 and 4.1.1 of the main paper for the explanation of the quantities shown in the table.

| Mechanism ID | Averaged error function value |  |  |  |  |  |  |  | Simulations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EST-IDTall | EJSRall | EFRall | ELBVall | Eaverageall | Etotalall | (D.p.) | (Efsd > 400) | Successful | Failed |
| POLIMI-2020 | 9.7 | 21.4 | 10.0 | 9.3 | 12.6 | 13.2 | (3292) | (84) | 3769 | 1 |
| Han-2020 | 5.9 | 26.4 | 13.7 | 9.5 | 13.9 | 15.4 | (3286) | (90) | 3769 | 1 |
| KAUST-2021 | 26.6 | 15.4 | 6.0 | 8.9 | 14.2 | 11.5 | (3301) | (74) | 3768 | 2 |
| Otomo-2018 | 5.8 | 22.8 | 10.2 | 20.8 | 14.9 | 17.7 | (3281) | (94) | 3768 | 2 |
| Mathieu-2015 | 5.7 | 21.2 | 9.2 | 27.5 | 15.9 | 19.4 | (3279) | (87) | 3759 | 11 |
| Mei-2020 | 33.8 | 21.3 | 10.3 | 8.9 | 18.6 | 14.9 | (3280) | (95) | 3769 | 1 |
| Gotama-2022 | 28.6 | 22.3 | 14.1 | 11.7 | 19.2 | 16.8 | (3292) | (84) | 3769 | 1 |
| Mei-2021 | 42.9 | 20.3 | 9.6 | 6.4 | 19.8 | 14.2 | (3279) | (95) | 3767 | 3 |
| Konnov-2021 | 23.3 | 26.3 | 17.5 | 13.7 | 20.2 | 19.2 | (3257) | (119) | 3769 | 1 |
| Shrestha-2021 | 22.8 | 36.2 | 10.2 | 12.5 | 20.5 | 20.1 | (3255) | (121) | 3769 | 1 |
| SanDiego-2018 | 45.4 | 28.6 | 16.1 | 14.4 | 26.1 | 21.5 | (3283) | (92) | 3768 | 2 |
| NUIG-2020 | 16.9 | 29.9 | 22.2 | 40.5 | 27.4 | 31.0 | (3268) | (108) | 3770 | 0 |
| Nakamura-2017 | 34.1 | 38.5 | 20.8 | 16.5 | 27.5 | 25.7 | (3247) | (129) | 3770 | 0 |
| Glarborg-2018 | 9.5 | 28.8 | 12.3 | 75.9 | 31.6 | 40.6 | (3280) | (96) | 3770 | 0 |
| All data points: | 200 | 1595 | 1125 | 850 |  |  | 3770 |  |  |  |
| Efsd > 9 with each model: | 2 (1.0%) | 238 (14.9%) | 139 (12.4%) | 15 (1.8%) |  |  | 394 (10.5%) |  |  |  |
| All data series: | 26 | 108 | 87 | 129 |  |  | 350 |  |  |  |
| All datasets: | 26 | 29 | 23 | 129 |  |  | 207 |  |  |  |

# Additional sensitivity analysis results

Results of the sensitivity analysis not shown in the main paper are summarized and briefly discussed here.

## NH3/H2 fuel mixtures

In Table A11, the overall results of the sensitivity analysis of thermodynamic properties are shown for 0D experiments. The reason of showing overall results is that the results for the different types of experiments (ST-IDT, JSR, and FR) are very similar. Results for LBV measurements are discussed in the main paper.

The most sensitive species are very similar for each thermodynamic parameter, and also, for each mechanism, only minor differences can be observed. The H/O radicals (H, O, OH, HO2) that govern the combustion of hydrogen-containing fuels are among the most sensitive species in POLIMI-2020 and KAUST-2021 for each thermodynamic property. The same is true for Shrestha-2021, except for the O radical.

NH3 is present as a fuel in all experiments; therefore, it is not surprising that it is among the three most sensitive species in all cases. The amino radical (NH2) is a key intermediate in ammonia oxidation, and accordingly, the sensitivity of its thermodynamic data is similar to that of NH3. In several experiments, the gas mixtures contain H2 as a co-fuel, so H2 is also among the most sensitive species in all cases. In all investigated experiments, O2 is the oxidizer, which explains its high sensitivity in all models. In POLIMI-2020 and KAUST-2021, it is among the three most sensitive species, but its snij value is significantly smaller in Shrestha-2021 than in the other two models for each thermodynamic parameter.

Table A12. Comparison of the overall sensitivity analysis results of the thermodynamic data for 0D experiments (ST-IDT, JSR, FR) with NH3/H2 fuel mixtures (2541 investigated data points). The 10-10 most sensitive species of the three investigated mechanisms are shown for each thermodynamic property separately.

| Zero-dimensional experiments with NH3/H2 fuel mixtures (2541 investigated data points) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| POLIMI-2020 |  |  | KAUST-2021 |  |  | Shrestha-2021 |  |  |
| Species | Freq.a | snijb | Species | Freq.a | snijb | Species | Freq.a | snijb |
| Constant-pressure standard molar heat capacity at 300 K |  |  |  |  |  |  |  |  |
| NH₂ | 83.9% | 0.556 | NH₂ | 90.7% | 0.676 | NH₂ | 81.6% | 0.630 |
| O₂ | 83.7% | 0.524 | NH₃ | 86.7% | 0.671 | H | 81.0% | 0.441 |
| NH₃ | 82.3% | 0.600 | O₂ | 75.9% | 0.451 | NH₃ | 76.1% | 0.559 |
| HO₂ | 73.5% | 0.472 | H | 69.5% | 0.379 | H₂ | 42.0% | 0.235 |
| H | 69.9% | 0.362 | HO₂ | 63.1% | 0.461 | O₂ | 40.3% | 0.198 |
| H₂ | 44.3% | 0.271 | H₂ | 52.3% | 0.301 | HO₂ | 33.9% | 0.157 |
| H₂NO | 39.3% | 0.125 | H₂NO | 32.8% | 0.139 | OH | 33.1% | 0.181 |
| O | 38.5% | 0.145 | N₂H₃ | 26.6% | 0.123 | N₂H₃ | 32.5% | 0.168 |
| HNOH | 29.9% | 0.104 | O | 25.5% | 0.112 | H₂O | 30.2% | 0.131 |
| OH | 29.1% | 0.137 | HNOH | 22.5% | 0.100 | H₂NO | 28.7% | 0.142 |
| Standard enthalpy of formation at 300 K (ssnij values instead of snij values)c |  |  |  |  |  |  |  |  |
| O₂ | 80.3% | 0.546 | NH₂ | 86.0% | 0.614 | H | 81.4% | 0.612 |
| NH₂ | 79.5% | 0.512 | NH₃ | 82.1% | 0.599 | NH₂ | 78.9% | 0.591 |
| NH₃ | 77.1% | 0.522 | H | 73.2% | 0.492 | NH₃ | 72.7% | 0.501 |
| H | 71.2% | 0.473 | O₂ | 72.7% | 0.497 | H₂ | 41.2% | 0.243 |
| HO₂ | 65.2% | 0.401 | HO₂ | 58.8% | 0.412 | O₂ | 40.9% | 0.202 |
| H₂ | 44.2% | 0.256 | H₂ | 52.1% | 0.281 | OH | 32.1% | 0.172 |
| O | 40.5% | 0.186 | O | 29.0% | 0.144 | HO₂ | 29.5% | 0.128 |
| H₂NO | 33.2% | 0.094 | H₂NO | 26.6% | 0.111 | N₂H₃ | 29.3% | 0.127 |
| OH | 27.9% | 0.133 | N₂H₃ | 21.9% | 0.088 | H₂O | 28.9% | 0.118 |
| HNOH | 24.2% | 0.080 | OH | 17.7% | 0.101 | O | 26.6% | 0.138 |
| Standard molar entropy at 300 K |  |  |  |  |  |  |  |  |
| O₂ | 83.3% | 0.584 | NH₂ | 88.8% | 0.647 | NH₂ | 80.5% | 0.628 |
| NH₂ | 81.4% | 0.540 | NH₃ | 84.8% | 0.619 | H | 78.6% | 0.416 |
| NH₃ | 78.5% | 0.543 | O₂ | 77.9% | 0.520 | NH₃ | 74.3% | 0.536 |
| HO₂ | 72.7% | 0.475 | H | 66.5% | 0.365 | O₂ | 42.2% | 0.225 |
| H | 66.6% | 0.342 | HO₂ | 62.7% | 0.478 | H₂ | 37.9% | 0.188 |
| O | 41.4% | 0.169 | H₂ | 47.9% | 0.240 | HO₂ | 34.5% | 0.167 |
| H₂ | 40.7% | 0.219 | H₂NO | 33.7% | 0.137 | OH | 32.7% | 0.181 |
| H₂NO | 38.4% | 0.115 | O | 29.7% | 0.133 | N₂H₃ | 32.0% | 0.161 |
| OH | 28.5% | 0.136 | N₂H₃ | 25.0% | 0.112 | H₂O | 29.4% | 0.126 |
| HNOH | 28.5% | 0.103 | HNOH | 22.2% | 0.097 | H₂NO | 29.0% | 0.144 |

a: Number of data points for which the thermodynamic property has snij ≥ 0.1 or ssnij ≥ 0.1c normalized by the total number of data points.

b: Average of the snij or ssnijc values over all included data points for each thermodynamic property.

c: Refer to Kovács et al. 91 for the definition of the ssnij values (scaled semi-normalized sensitivity coefficients).

## NH3/syngas fuel mixtures

Tables A12 and A13 show the results of the sensitivity analysis of thermochemical parameters for concentration and LBV measurements, respectively. Results for LBV measurements are discussed in the main paper.

In the case of 0D experiments, simulation results are much less sensitive to the thermochemical properties of NH3 and NH2 than in the case of NH3/H2 fuel mixtures (Table A11). This can be explained by the fact that in JSR experiments (Ding et al. 11), the fuel is an NH3/H2/CO mixture, and the molar ratio of NH3 in the fuel is as small as 1/11. In addition, in FR experiments (Wargadalam et al. 12), the fuel is an NH3/CO gas mixture, in which the molar ratio of NH3 is only 1/6, and the gas mixture is very oxidative (φ = 0.01).

Table A13. Comparison of the overall sensitivity analysis results of thermodynamic data for concentration measurements in JSRs and FRs using NH3/syngas fuel mixtures (202 investigated data points). The 10-10 most sensitive species of the three investigated mechanisms are shown for each thermodynamic property separately.

| Concentration measurements in JSRs and FRs: 202 investigated data points |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mei-2021 |  |  | Mei-2020 |  |  | Shrestha-2021 |  |  |
| Species | Freq.a | snijb | Species | Freq.a | snijb | Species | Freq.a | snijb |
| Constant-pressure standard molar heat capacity at 300 K |  |  |  |  |  |  |  |  |
| O₂ | 88.6% | 0.518 | H | 90.1% | 0.447 | H | 90.6% | 0.375 |
| H | 84.2% | 0.377 | O₂ | 86.1% | 0.493 | OH | 87.6% | 0.605 |
| OH | 73.3% | 0.488 | OH | 72.8% | 0.481 | O₂ | 87.6% | 0.498 |
| O | 70.3% | 0.260 | H₂O | 67.8% | 0.249 | H₂O | 86.1% | 0.343 |
| H₂O | 69.3% | 0.293 | NH₂ | 56.4% | 0.380 | CO | 68.3% | 0.361 |
| CO₂ | 54.0% | 0.349 | CO₂ | 54.0% | 0.338 | CO₂ | 60.9% | 0.407 |
| CO | 53.0% | 0.283 | CO | 52.5% | 0.276 | O | 45.0% | 0.255 |
| NO | 41.1% | 0.193 | O | 48.5% | 0.155 | H₂ | 32.2% | 0.204 |
| NH | 41.1% | 0.163 | HNO | 41.1% | 0.185 | NH₂ | 25.7% | 0.184 |
| H₂ | 33.7% | 0.212 | NH | 40.6% | 0.169 | H₂NO | 18.8% | 0.069 |
| Standard enthalpy of formation at 300 K (ssnij values instead of snij values) c |  |  |  |  |  |  |  |  |
| H | 89.1% | 0.515 | H | 94.1% | 0.589 | H | 93.1% | 0.524 |
| O₂ | 86.6% | 0.518 | O₂ | 83.7% | 0.492 | OH | 89.6% | 0.592 |
| O | 76.7% | 0.350 | OH | 74.8% | 0.459 | O₂ | 86.1% | 0.494 |
| OH | 75.2% | 0.476 | O | 65.3% | 0.200 | H₂O | 84.7% | 0.312 |
| H₂O | 67.8% | 0.265 | H₂O | 62.4% | 0.227 | CO | 67.3% | 0.405 |
| CO | 53.0% | 0.305 | CO | 52.5% | 0.295 | CO₂ | 59.9% | 0.364 |
| CO₂ | 52.5% | 0.306 | CO₂ | 51.5% | 0.294 | O | 58.9% | 0.317 |
| NO | 42.1% | 0.179 | NH₂ | 50.5% | 0.334 | H₂ | 32.2% | 0.204 |
| NH | 42.1% | 0.162 | NH | 40.6% | 0.162 | NH₂ | 25.7% | 0.179 |
| H₂ | 33.2% | 0.216 | H₂ | 39.6% | 0.228 | H₂NO | 14.4% | 0.058 |
| Standard molar entropy at 300 K |  |  |  |  |  |  |  |  |
| O₂ | 89.6% | 0.577 | O₂ | 88.6% | 0.555 | O₂ | 90.1% | 0.555 |
| H | 83.2% | 0.350 | H | 88.6% | 0.419 | H | 88.6% | 0.340 |
| O | 74.8% | 0.301 | OH | 73.3% | 0.487 | OH | 87.6% | 0.608 |
| OH | 73.3% | 0.493 | H₂O | 66.3% | 0.242 | H₂O | 84.7% | 0.329 |
| H₂O | 68.3% | 0.277 | O | 60.9% | 0.190 | CO | 67.8% | 0.411 |
| CO₂ | 55.0% | 0.344 | NH₂ | 55.9% | 0.361 | CO₂ | 60.4% | 0.402 |
| CO | 55.0% | 0.318 | CO₂ | 53.5% | 0.335 | O | 57.9% | 0.285 |
| NO | 42.6% | 0.209 | CO | 53.5% | 0.313 | H₂ | 29.7% | 0.166 |
| NH | 41.6% | 0.161 | HNO | 41.6% | 0.183 | NH₂ | 25.7% | 0.180 |
| H₂ | 31.7% | 0.171 | NH | 40.6% | 0.169 | H₂NO | 19.3% | 0.070 |

a: Number of data points for which the thermodynamic property has snij ≥ 0.1 or ssnij ≥ 0.1 divided (normalized) by the total number of data points.

b: Average of the snij or ssnij values over all included data points for each thermodynamic property.

c: Refer to Kovács et al. 91 for the definition of the ssnij values (scaled semi-normalized sensitivity coefficients).

Table A14. Comparison of the overall sensitivity analysis results of thermodynamic data for laminar burning velocity measurements using NH3/syngas fuel mixtures (874 investigated data points). The 10-10 most sensitive species of the three investigated mechanisms are shown for each thermodynamic property separately.

| LBV experiments: 874 investigated data points |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mei-2021 |  |  | Mei-2020 |  |  | Shrestha-2021 |  |  |
| Species | Freq.a | snijb | Species | Freq.a | snijb | Species | Freq.a | snijb |
| Constant-pressure standard molar heat capacity at 300 K |  |  |  |  |  |  |  |  |
| N₂ | 97.7% | 0.318 | N₂ | 97.9% | 0.329 | N₂ | 97.6% | 0.315 |
| H | 96.2% | 0.810 | H | 95.3% | 0.793 | H | 94.2% | 0.771 |
| H₂ | 92.9% | 0.494 | H₂ | 93.4% | 0.511 | H₂ | 93.8% | 0.642 |
| CO₂ | 91.1% | 0.498 | CO₂ | 91.4% | 0.520 | CO₂ | 91.3% | 0.568 |
| CO | 90.5% | 0.437 | CO | 90.8% | 0.454 | CO | 89.6% | 0.488 |
| H₂O | 89.8% | 0.419 | H₂O | 87.9% | 0.407 | H₂O | 84.1% | 0.308 |
| OH | 76.0% | 0.501 | OH | 80.3% | 0.500 | OH | 81.4% | 0.415 |
| O₂ | 65.2% | 0.238 | HNO | 78.4% | 0.202 | NH₃ | 71.6% | 0.294 |
| NH₂ | 61.6% | 0.210 | O₂ | 65.7% | 0.259 | NH₂ | 67.8% | 0.260 |
| NNH | 61.1% | 0.117 | NH₂ | 64.9% | 0.189 | O₂ | 62.0% | 0.219 |
| Standard enthalpy of formation at 300 K (ssnij values instead of snij values) c |  |  |  |  |  |  |  |  |
| O₂ | 100.0% | 0.542 | O₂ | 100.0% | 0.574 | O₂ | 96.1% | 0.487 |
| H | 96.8% | 0.853 | H | 96.2% | 0.848 | H | 91.5% | 0.791 |
| H₂O | 92.7% | 0.468 | H₂O | 93.5% | 0.500 | H₂ | 91.2% | 0.527 |
| NH₃ | 92.0% | 0.370 | NH₃ | 93.4% | 0.395 | H₂O | 88.1% | 0.554 |
| H₂ | 91.9% | 0.417 | H₂ | 92.4% | 0.434 | CO | 85.9% | 0.457 |
| CO₂ | 89.4% | 0.424 | CO₂ | 89.2% | 0.447 | CO₂ | 85.7% | 0.461 |
| CO | 89.4% | 0.420 | CO | 89.2% | 0.442 | N₂ | 81.1% | 0.161 |
| N₂ | 82.4% | 0.158 | N₂ | 83.1% | 0.166 | NH₃ | 77.8% | 0.192 |
| OH | 66.2% | 0.435 | OH | 70.8% | 0.436 | OH | 71.6% | 0.355 |
| NH₂ | 53.5% | 0.144 | HNO | 65.9% | 0.126 | NH₂ | 58.8% | 0.182 |
| Standard molar entropy at 300 K |  |  |  |  |  |  |  |  |
| H | 95.8% | 0.795 | H | 95.0% | 0.781 | CO₂ | 92.6% | 0.582 |
| CO₂ | 93.5% | 0.523 | CO₂ | 93.6% | 0.543 | H | 92.4% | 0.752 |
| CO | 92.6% | 0.482 | CO | 92.6% | 0.499 | CO | 92.0% | 0.531 |
| H₂ | 88.3% | 0.388 | H₂ | 89.1% | 0.402 | H₂ | 90.2% | 0.508 |
| H₂O | 82.5% | 0.325 | H₂O | 82.8% | 0.321 | H₂O | 82.3% | 0.303 |
| OH | 77.5% | 0.519 | OH | 81.7% | 0.517 | OH | 82.2% | 0.436 |
| O₂ | 67.8% | 0.270 | HNO | 79.5% | 0.231 | NH₃ | 68.6% | 0.286 |
| NNH | 63.4% | 0.131 | O₂ | 68.4% | 0.293 | NH₂ | 66.9% | 0.277 |
| NH₂ | 61.3% | 0.220 | NH₂ | 65.2% | 0.196 | O₂ | 65.3% | 0.249 |
| NO | 61.1% | 0.138 | O | 64.4% | 0.140 | HO₂ | 55.7% | 0.200 |

a: Number of data points for which the thermodynamic property has snij ≥ 0.1 or ssnij ≥ 0.1 divided (normalized) by the total number of data points.

b: Average of the snij or ssnij values over all included data points for each thermodynamic property.

c: Refer to Kovács et al. 91 for the definition of the ssnij values (scaled semi-normalized sensitivity coefficients).

References

1. ELTE Chemical Kinetics Laboratory, MTA-ELTE Complex Chemical Systems Research Group. ReSpecTh Information System. https://respecth.hu/;  [accessed 22 February 2023].

2. Chen J, Jiang X, Qin X, Huang Z. Effect of hydrogen blending on the high temperature auto-ignition of ammonia at elevated pressure. Fuel 2021;287:119563. https://doi.org/10.1016/j.fuel.2020.119563.

3. Mathieu O, Petersen EL. Experimental and modeling study on the high-temperature oxidation of Ammonia and related NOx chemistry. Combust. Flame 2015;162(3):554–70. https://doi.org/10.1016/j.combustflame.2014.08.022.

4. Shu B, Vallabhuni SK, He X, Issayev G, Moshammer K, Farooq A, et al. A shock tube and modeling study on the autoignition properties of ammonia at intermediate temperatures. Proc. Combust. Inst. 2019;37(1):205–11. https://doi.org/10.1016/j.proci.2018.07.074.

5. Zhang X, Moosakutty SP, Rajan RP, Younes M, Sarathy SM. Combustion chemistry of ammonia/hydrogen mixtures: Jet-stirred reactor measurements and comprehensive kinetic modeling. Combust. Flame 2021;234:111653. https://doi.org/10.1016/j.combustflame.2021.111653.

6. Osipova KN, Zhang X, Sarathy SM, Korobeinichev OP, Shmakov AG. Ammonia and ammonia/hydrogen blends oxidation in a jet-stirred reactor: Experimental and numerical study. Fuel 2022;310:122202. https://doi.org/10.1016/j.fuel.2021.122202.

7. Stagni A, Cavallotti C, Arunthanayothin S, Song Y, Herbinet O, Battin-Leclerc F, et al. An experimental, theoretical and kinetic-modeling study of the gas-phase oxidation of ammonia. React. Chem. Eng. 2020;5(4):696–711. https://doi.org/10.1039/c9re00429g.

8. Dagaut P. On the Oxidation of Ammonia and Mutual Sensitization of the Oxidation of NO and Ammonia: Experimental and Kinetic Modeling. Combust. Sci. Technol. 2022;194(1):117–29. https://doi.org/10.1080/00102202.2019.1678380.

9. Manna MV, Sabia P, Ragucci R, De Joannon M. Oxidation and pyrolysis of ammonia mixtures in model reactors. Fuel 2020;264:116768. https://doi.org/10.1016/j.fuel.2019.116768.

10. Sabia P, Manna MV, Cavaliere A, Ragucci R, De Joannon M. Ammonia oxidation features in a Jet Stirred Flow Reactor. The role of NH2 chemistry. Fuel 2020;276:118054. https://doi.org/10.1016/j.fuel.2020.118054.

11. Ding C, Li P, Wang K, Shi G, Wang F, Liu Z. Experimental and Kinetic Study on the Oxidation of Syngas-Ammonia under Both N2 and CO2 Atmospheres in a Jet-Stirred Reactor. Energy Fuels 2021;35(14):11445–56. https://doi.org/10.1021/acs.energyfuels.1c00647.

12. Wargadalam VJ, Löffler G, Winter F, Hofbauer H. Homogeneous formation of NO and N2O from the oxidation of HCN and NH3 at 600–1000 °C. Combust. Flame 2000;120(4):465–78. https://doi.org/10.1016/S0010-2180(99)00107-8.

13. Hulgaard T, Dam‐Johansen K. Homogeneous nitrous oxide formation and destruction under combustion conditions. AIChE J. 1993;39(8):1342–54. https://doi.org/10.1002/aic.690390811.

14. Song Y, Hashemi H, Christensen JM, Zou C, Marshall P, Glarborg P. Ammonia oxidation at high pressure and intermediate temperatures. Fuel 2016;181:358–65. https://doi.org/10.1016/j.fuel.2016.04.100.

15. Caton JA, Narney JK, Cariappa HC, Laster WR. The selective non‐catalytic reduction of nitric oxide using ammonia at up to 15% oxygen. Can. J. Chem. Eng. 1995;73(3):345–50. https://doi.org/10.1002/cjce.5450730311.

16. Dean AM, Hardy JE, Lyon RK. Kinetics and mechanism of NH3 oxidation. Symposium (International) on Combustion 1982;19(1):97–105. https://doi.org/10.1016/S0082-0784(82)80182-3.

17. Abián M, Benés M, Goñi AD, Muñoz B, Alzueta MU. Study of the oxidation of ammonia in a flow reactor. Experiments and kinetic modeling simulation. Fuel 2021;300:120979. https://doi.org/10.1016/j.fuel.2021.120979.

18. Lee JH, Lee SI, Kwon OC. Effects of ammonia substitution on hydrogen/air flame propagation and emissions. Int. J. Hydrog. Energy 2010;35(20):11332–41. https://doi.org/10.1016/j.ijhydene.2010.07.104.

19. Lee JH, Kim JH, Park JH, Kwon OC. Studies on properties of laminar premixed hydrogen-added ammonia/air flames for hydrogen production. Int. J. Hydrog. Energy 2010;35(3):1054–64. https://doi.org/10.1016/j.ijhydene.2009.11.071.

20. Ichikawa A, Hayakawa A, Kitagawa Y, Kunkuma Amila Somarathne KD, Kudo T, Kobayashi H. Laminar burning velocity and Markstein length of ammonia/hydrogen/air premixed flames at elevated pressures. Int. J. Hydrog. Energy 2015;40(30):9570–8. https://doi.org/10.1016/j.ijhydene.2015.04.024.

21. Han X, Wang Z, Costa M, Sun Z, He Y, Cen K. Experimental and kinetic modeling study of laminar burning velocities of NH3/air, NH3/H2/air, NH3/CO/air and NH3/CH4/air premixed flames. Combust. Flame 2019;206:214–26. https://doi.org/10.1016/j.combustflame.2019.05.003.

22. Wang S, Wang Z, Elbaz AM, Han X, He Y, Costa M, et al. Experimental study and kinetic analysis of the laminar burning velocity of NH3/syngas/air, NH3/CO/air and NH3/H2/air premixed flames at elevated pressures. Combust. Flame 2020;221:270–87. https://doi.org/10.1016/j.combustflame.2020.08.004.

23. Lhuillier C, Brequigny P, Lamoureux N, Contino F, Mounaïm-Rousselle C. Experimental investigation on laminar burning velocities of ammonia/hydrogen/air mixtures at elevated temperatures. Fuel 2020;263:116653. https://doi.org/10.1016/j.fuel.2019.116653.

24. Shrestha KP, Lhuillier C, Barbosa AA, Brequigny P, Contino F, Mounaïm-Rousselle C, et al. An experimental and modeling study of ammonia with enriched oxygen content and ammonia/hydrogen laminar flame speed at elevated pressure and temperature. Proc. Combust. Inst. 2021;38(2):2163–74. https://doi.org/10.1016/j.proci.2020.06.197.

25. Gotama GJ, Hayakawa A, Okafor EC, Kanoshima R, Hayashi M, Kudo T, et al. Measurement of the laminar burning velocity and kinetics study of the importance of the hydrogen recovery mechanism of ammonia/hydrogen/air premixed flames. Combust. Flame 2022;236:111753. https://doi.org/10.1016/j.combustflame.2021.111753.

26. Mei B, Zhang J, Shi X, Xi Z, Li Y. Enhancement of ammonia combustion with partial fuel cracking strategy: Laminar flame propagation and kinetic modeling investigation of NH3/H2/N2/air mixtures up to 10 atm. Combust. Flame 2021;231:111472. https://doi.org/10.1016/j.combustflame.2021.111472.

27. Wang N, Huang S, Zhang Z, Li T, Yi P, Wu D, et al. Laminar burning characteristics of ammonia/hydrogen/air mixtures with laser ignition. Int. J. Hydrog. Energy 2021;46(62):31879–93. https://doi.org/10.1016/j.ijhydene.2021.07.063.

28. Osipova KN, Korobeinichev OP, Shmakov AG. Chemical structure and laminar burning velocity of atmospheric pressure premixed ammonia/hydrogen flames. Int. J. Hydrog. Energy 2021;46(80):39942–54. https://doi.org/10.1016/j.ijhydene.2021.09.188.

29. Hayakawa A, Goto T, Mimoto R, Arakawa Y, Kudo T, Kobayashi H. Laminar burning velocity and Markstein length of ammonia/air premixed flames at various pressures. Fuel 2015;159:98–106. https://doi.org/10.1016/j.fuel.2015.06.070.

30. Liu Q, Chen X, Huang J, Shen Y, Zhang Y, Liu Z. The characteristics of flame propagation in ammonia/oxygen mixtures. J. Hazard. Mater. 2019;363:187–96. https://doi.org/10.1016/j.jhazmat.2018.09.073.

31. Mei B, Zhang X, Ma S, Cui M, Guo H, Cao Z, et al. Experimental and kinetic modeling investigation on the laminar flame propagation of ammonia under oxygen enrichment and elevated pressure conditions. Combust. Flame 2019;210:236–46. https://doi.org/10.1016/j.combustflame.2019.08.033.

32. Wang D, Ji C, Wang Z, Wang S, Zhang T, Yang J. Measurement of oxy-ammonia laminar burning velocity at normal and elevated temperatures. Fuel 2020;279:118425. https://doi.org/10.1016/j.fuel.2020.118425.

33. Takeishi H, Hayashi J, Kono S, Arita W, Iino K, Akamatsu F. Characteristics of ammonia/N2/O2 laminar flame in oxygen-enriched air condition. Transactions of the JSME (in Japanese) 2015;81(824):414­–23. https://doi.org/10.1299/transjsme.14-00423.

34. Han X, Wang Z, He Y, Zhu Y, Cen K. Experimental and kinetic modeling study of laminar burning velocities of NH3/syngas/air premixed flames. Combust. Flame 2020;213:1–13. https://doi.org/10.1016/j.combustflame.2019.11.032.

35. Mei B, Ma S, Zhang Y, Zhang X, Li W, Li Y. Exploration on laminar flame propagation of ammonia and syngas mixtures up to 10 atm. Combust. Flame 2020;220:368–77. https://doi.org/10.1016/j.combustflame.2020.07.011.

36. Zhou S, Yang W, Tan H, An Q, Wang J, Dai H, et al. Experimental and kinetic modeling study on NH3/syngas/air and NH3/bio-syngas/air premixed laminar flames at elevated temperature. Combust. Flame 2021;233:111594. https://doi.org/10.1016/j.combustflame.2021.111594.

37. Yin G, Wang C, Zhou M, Zhou Y, Hu E, Huang Z. Experimental and kinetic study on laminar flame speeds of ammonia/syngas/air at a high temperature and elevated pressure. Front. Energ. 2021 https://doi.org/10.1007/s11708-021-0791-7.

38. Fujii N, Miyama H, Koshi M, Asaba T. Kinetics of ammonia oxidation in shock waves. Proc. Combust. Inst. 1981;18:873–83. https://doi.org/10.1016/S0082-0784(81)80091-4.

39. Dove JE, Nip WS. A shock-tube study of ammonia pyrolysis. Can. J. Chem. 1979;57:689–701. https://doi.org/10.1139/v79-112.

40. Drummond LJ. High temperature oxidation of ammonia. Combust. Sci. Technol. 1972;5(1):175–82. https://doi.org/10.1080/00102207208952518.

41. Bull DC. A shock tube study of the oxidation of ammonia. Combust. Flame 1968;12:603–10. https://doi.org/10.1016/0010-2180(68)90079-5.

42. Bradley JN, Butlin RN, Lewis D. Oxidation of ammonia in shock waves. Trans. Faraday Soc. 1968;64:71–8. http://dx.doi.org/10.1039/TF9686400071.

43. Miyama H. Kinetic Studies of Ammonia Oxidation in Shock Waves. IV. Comparison of Induction Periods for the Ignition of NH3-O2-N2 with Those for NH3-O2-Ar Mixtures. Bull. Chem. Soc. Jpn. 1968;41(8):1761–5. https://doi.org/10.1246/bcsj.41.1761.

44. Miyama H. Ignition of Ammonia–Oxygen Mixtures by Shock Waves. J. Chem. Phys. 1968;48(3):1421–2. https://doi.org/10.1063/1.1668832.

45. Miyama H, Endoh R. Ignition of ammonia-air mixtures by reflected shock waves. Combust. Flame 1967;11(4):359–60. https://doi.org/10.1016/0010-2180(67)90027-2.

46. Miyama H, Endoh R. Vibrational Relaxation of Nitrogen in Shock‐Heated NH3–O2–N2 Mixtures. J. Chem. Phys. 1967;46(5):2011–2. https://doi.org/10.1063/1.1840982.

47. Takeyama T, Miyama H. A shock-tube study of the ammonia-oxygen reaction. Symposium (International) on Combustion 1967;11(1):845–52. https://doi.org/10.1016/S0082-0784(67)80210-8.

48. Takeyama T, Miyama H. Kinetic Studies of Ammonia Oxidation in Shock Waves. I. The Reaction Mechanism for the Induction Period. Bull. Chem. Soc. Jpn. 1965;38(10):1670–4. https://doi.org/10.1246/bcsj.38.1670.

49. Takeyama T, Miyama H. Reaction Mechanism of Ammonia Oxidation in Shock Waves. J. Chem. Phys. 1965;42(10):3737–8. https://doi.org/10.1063/1.1695800.

50. Mathieu O, Kopp MM, Petersen EL. Shock-tube study of the ignition of multi-component syngas mixtures with and without ammonia impurities. Proc. Combust. Inst. 2013;34(2):3211–8. https://doi.org/10.1016/j.proci.2012.05.008.

51. Nakamura H, Hasegawa S. Combustion and ignition characteristics of ammonia/air mixtures in a micro flow reactor with a controlled temperature profile. Proc. Combust. Inst. 2017;36(3):4217–26. https://doi.org/10.1016/j.proci.2016.06.153.

52. Nakamura H, Hasegawa S, Tezuka T. Kinetic modeling of ammonia/air weak flames in a micro flow reactor with a controlled temperature profile. Combust. Flame 2017;185:16–27. https://doi.org/10.1016/j.combustflame.2017.06.021.

53. Kumar P, Meyer TR. Experimental and modeling study of chemical-kinetics mechanisms for H2–NH3–air mixtures in laminar premixed jet flames. Fuel 2013;108:166–76. https://doi.org/10.1016/j.fuel.2012.06.103.

54. Li J, Huang H, Kobayashi N, He Z, Nagai Y. Study on using hydrogen and ammonia as fuels: Combustion characteristics and NOx formation. Int. J. Energ. Res. 2014;38(9):1214–23. https://doi.org/10.1002/er.3141.

55. Takizawa K, Takahashi A, Tokuhashi K, Kondo S, Sekiya A. Burning velocity measurements of nitrogen-containing compounds. J. Hazard. Mater. 2008;155(1-2):144–52. https://doi.org/10.1016/j.jhazmat.2007.11.089.

56. Jabbour T, Clodic DF. Burning Velocity and Refrigerant Flammability Classification. ASHRAE Tran. 2004;110:522–33.

57. Pfahl UJ, Ross MC, Shepherd JE, Pasamehmetoglu KO, Unal C. Flammability limits, ignition energy, and flame speeds in H2–CH4–NH3–N2O–O2–N2 mixtures. Combust. Flame 2000;123(1-2):140–58. https://doi.org/10.1016/S0010-2180(00)00152-8.

58. Ronney PD. Effect of Chemistry and Transport Properties on Near-Limit Flames at Microgravity. Combust. Sci. Technol. 1988;59(1-3):123–41. https://doi.org/10.1080/00102208808947092.

59. Zakaznov VF, Kursheva LA, Fedina ZI. Determination of normal flame velocity and critical diameter of flame extinction in ammonia-air mixture. Combust. Explos. Shock Waves 1978;14(6):710–3. https://doi.org/10.1007/BF00786097.

60. Davis SG, Pagliaro JL, Debold TF, Van Wingerden M, Van Wingerden K. Flammability and explosion characteristics of mildly flammable refrigerants. Journal of Loss Prevention in the Process Industries 2017;49:662–74. https://doi.org/10.1016/j.jlp.2017.05.019.

61. Chaos M, Dryer FL. Chemical-kinetic modeling of ignition delay: Considerations in interpreting shock tube data. Int. J. Chem. Kinet. 2010;42(3):143–50. https://doi.org/10.1002/kin.20471.

62. Petersen EL, Hanson RK. Nonideal effects behind reflected shock waves in a high-pressure shock tube. Shock Waves 2001;10(6):405–20. https://doi.org/10.1007/pl00004051.

63. Nagy T, Minimal Spline Fit Introducing Root-Mean-Square Fitting of Data Series with Akima Splines. January 5 (2020) version http://respecth.elte.hu/compProgs.php [accessed 18 January 2023].

64. Nagy T, Turányi T, Minimal Spline Fit: a model-free method for determining statistical noise of experimental data series. European Combustion Meeting; 2021; 14-15 April; Naples, Italy. Paper 336. http://garfield.chem.elte.hu/Turanyi/pdf/153_Nagy_ECM21-336.pdf [accessed 18 January 2023].

65. Olm C, Zsély IG, Varga T, Curran HJ, Turányi T. Comparison of the performance of several recent syngas combustion mechanisms. Combust. Flame 2015;162(5):1793–812. https://doi.org/10.1016/j.combustflame.2014.12.001.

66. Zhang P, Zsély IG, Samu V, Nagy T, Turányi T. Comparison of Methane Combustion Mechanisms Using Shock Tube and Rapid Compression Machine Ignition Delay Time Measurements. Energy Fuels 2021;35(15):12329–51. https://doi.org/10.1021/acs.energyfuels.0c04277.

67. Kawka L, Juhász G, Papp M, Nagy T, Zsély IG, Turányi T. Comparison of detailed reaction mechanisms for homogeneous ammonia combustion. Z. Phys. Chem. 2020;234(7-9):1329–57. https://doi.org/10.1515/zpch-2020-1649.

68. Zhang P, Zsély IG, Papp M, Nagy T, Turányi T. Comparison of methane combustion mechanisms using laminar burning velocity measurements. Combust. Flame 2022;238:111867. https://doi.org/10.1016/j.combustflame.2021.111867.

69. Kéromnès A, Metcalfe WK, Heufer KA, Donohoe N, Das AK, Sung C-J, et al. An experimental and detailed chemical kinetic modeling study of hydrogen and syngas mixture oxidation at elevated pressures. Combust. Flame 2013;160(6):995–1011. https://doi.org/10.1016/j.combustflame.2013.01.001.

70. Tian Z, Li Y, Zhang L, Glarborg P, Qi F. An experimental and kinetic modeling study of premixed NH3/CH4/O2/Ar flames at low pressure. Combust. Flame 2009;156(7):1413–26. https://doi.org/10.1016/j.combustflame.2009.03.005.

71. Lamoureux N, Merhubi HE, Pillier L, De Persis S, Desgroux P. Modeling of NO formation in low pressure premixed flames. Combust. Flame 2016;163:557−75. https://doi.org/10.1016/j.combustflame.2015.11.007.

72. Mechanical and Aerospace Engineering (Combustion Research), University of California at San Diego. Chemical-Kinetic Mechanisms for Combustion Applications. http://web.eng.ucsd.edu/mae/groups/combustion/mechanism.html;  [accessed 22 February 2023].

73. Otomo J, Koshi M, Mitsumori T, Iwasaki H, Yamada K. Chemical kinetic modeling of ammonia oxidation with improved reaction mechanism for ammonia/air and ammonia/hydrogen/air combustion. Int. J. Hydrog. Energy 2018;43(5):3004–14. https://doi.org/10.1016/j.ijhydene.2017.12.066.

74. Glarborg P, Miller JA, Ruscic B, Klippenstein SJ. Modeling nitrogen chemistry in combustion. Prog. Energy Combust. Sci. 2018;67:31–68. https://doi.org/10.1016/j.pecs.2018.01.002.

75. Okafor EC, Naito Y, Colson S, Ichikawa A, Kudo T, Hayakawa A, et al. Experimental and numerical study of the laminar burning velocity of CH4–NH3–air premixed flames. Combust. Flame 2018;187:185–98. https://doi.org/10.1016/j.combustflame.2017.09.002.

76. The CRECK Modeling Group (Politecnico di Milano). NH3 mechanism (Version 2003, March 2020). http://creckmodeling.chem.polimi.it/menu-kinetics/menu-kinetics-special-mechanisms/menu-kinetics-nh3-mechanism; 2020. [accessed 22 February 2023].

77. Wang Q-D, Sun Y, Curran HJ. Comparative Chemical Kinetic Analysis and Skeletal Mechanism Generation for Syngas Combustion with NOx Chemistry. Energy Fuels 2020;34(1):949−64. https://doi.org/10.1021/acs.energyfuels.9b03440.

78. Lavadera ML, Konnov AA. Laminar burning velocities of methane + formic acid + air flames: Experimental and modeling study. Combust. Flame 2021;225:65–73. https://doi.org/10.1016/j.combustflame.2020.10.050.

79. Mendiara T, Glarborg P. Ammonia chemistry in oxy-fuel combustion of methane. Combust. Flame 2009;156(10):1937–49. https://doi.org/10.1016/j.combustflame.2009.07.006.

80. Mével R, Javoy S, Lafosse F, Chaumeix N, Dupré G, Paillard C-E. Hydrogen–nitrous oxide delay times: Shock tube experimental study and kinetic modelling. Proc. Combust. Inst. 2009;32(1):359–66. https://doi.org/10.1016/j.proci.2008.06.171.

81. Klippenstein SJ, Harding LB, Glarborg P, Miller JA. The role of NNH in NO formation and control. Combust. Flame 2011;158(4):774–89. https://doi.org/10.1016/j.combustflame.2010.12.013.

82. Abian M, Alzueta MU, Glarborg P. Formation of NO from N2/O2 Mixtures in a Flow Reactor: Toward an Accurate Prediction of Thermal NO. Int. J. Chem. Kinet. 2015;47(8):518–32. https://doi.org/10.1002/kin.20929.

83. Zhang Y, Mathieu O, Petersen EL, Bourque G, Curran HJ. Assessing the predictions of a NOx kinetic mechanism on recent hydrogen and syngas experimental data. Combust. Flame 2017;182:122–41. https://doi.org/10.1016/j.combustflame.2017.03.019.

84. Klippenstein SJ, Pfeifle M, Jasper AW, Glarborg P. Theory and modeling of relevance to prompt-NO formation at high pressure. Combust. Flame 2018;195:3–17. https://doi.org/10.1016/j.combustflame.2018.04.029.

85. Shrestha KP, Seidel L, Zeuch T, Mauss F. Detailed Kinetic Mechanism for the Oxidation of Ammonia Including the Formation and Reduction of Nitrogen Oxides. Energy Fuels 2018;32(10):10202−17. https://doi.org/10.1021/acs.energyfuels.8b01056.

86. Mei B, Ma S, Zhang X, Li Y. Characterizing ammonia and nitric oxide interaction with outwardly propagating spherical flame method. Proc. Combust. Inst. 2021;38(2):2477–85. https://doi.org/10.1016/j.proci.2020.07.133.

87. Duynslaegher C, Contino F, Vandooren J, Jeanmart H. Modeling of ammonia combustion at low pressure. Combust. Flame 2012;159(9):2799–805. http://dx.doi.org/10.1016/j.combustflame.2012.06.003.

88. Okafor EC, Naito Y, Colson S, Ichikawa A, Kudo T, Hayakawa A, et al. Measurement and modelling of the laminar burning velocity of methane-ammonia-air flames at high pressures using a reduced reaction mechanism. Combust. Flame 2019;204:162–75. https://doi.org/10.1016/j.combustflame.2019.03.008.

89. Bertolino A, Fürst M, Stagni A, Frassoldati A, Pelucchi M, Cavallotti C, et al. An evolutionary, data-driven approach for mechanism optimization: theory and application to ammonia combustion. Combust. Flame 2021;229:111366. https://doi.org/10.1016/j.combustflame.2021.02.012.

90. Han X, Lubrano Lavadera M, Konnov AA. An experimental and kinetic modeling study on the laminar burning velocity of NH3 + N2O + air flames. Combust. Flame 2021;228:13–28. https://doi.org/10.1016/j.combustflame.2021.01.027.

91. Kovács M, Papp M, Zsély IG, Turányi T. Main sources of uncertainty in recent methanol/NOx combustion models. Int. J. Chem. Kinet. 2021;53(7):884–900. https://doi.org/10.1002/kin.21490.